AlphaServer DS20 Pedestal–Rackmount Windows NT



[image: image1.png]



Compaq AlphaServer DS20 Pedestal–Rackmount
Windows NT
V1.1—16 February 1999
Product Description 
The Compaq AlphaServer DS20 is a 500MHz EV6 based  Alpha microprocessor server with 4 MB level two cache.  The AlphaServer DS20 supports two CPUs, and is available in Pedestal configurations with Windows NT operating system.  Pedestal configurations can be installed in M-series cabinet enclosures.  The AlphaServer DS20 system’s high-reliability features include: hot swap disk drives, redundant power supplies, ECC memory, heat sensor, fan failure sensor, and power supply sensor.  Integrated on the system motherboard are, a diskette controller, two serial ports, one parallel port, and keyboard and mouse controller.

AlphaServer DS20 Pedestal System supports up to 10 internal storage devices including one floppy diskette drive, one CD-ROM, one additional 5.25-inch removable media device, and seven hot swap StorageWorks disk drives.  The system supports up to 128 GB of internal storage with seven 18.2 GB disk drives.

System Ordering Rules
· AlphaServer DS20 systems require the mandatory selection of:  a system disk, two SCSI ports, and a network adapter.  Users can select 4.3 GB, 
9.1 GB, or 18.2 GB disk drive as the system disk.

· To order a complete system select:  (1) a system, (2) a system disk, (3) two SCSI ports, and (4) an Ethernet Adapter.

M-series Cabinet Enclosures
· Cabinet enclosures are available in two sizes: 67-inch and 79-inch.

· See Cabinet Configuration Chart for maximum combinations of systems and StorageWorks shelves supported in M-series Cabinet Enclosures.

Compaq and the names of Compaq products referenced herein are either trademarks and/or service marks or registered trademarks and/or service marks of Compaq.

DIGITAL is a Trademark of Compaq Computer Corporation. 

Microsoft, Windows, Windows NT, SQL Server, Office and BackOffice are either trademarks or registered trademarks of Microsoft Corporation.

Intel and Pentium are registered trademarks of Intel Corporation.

Other product and company names mentioned herein may be trademarks and/or service marks of their respective owners.

Step 1—AlphaServer DS20 Pedestal Systems

Configuration menus require the mandatory selection of:

· Windows NT Packaged System and mandatory components:

· One UltraSCSI system disk drive

· Minimum of two SCSI ports1
· One Ethernet adapter

· One graphics adapter

· Windows NT Server license, media kit from Step 11

· Country-specific keyboard and power cord from Step 10 (non-North American orders only)

1.
Two SCSI ports are required for internal removable media devices, and internal StorageWorks disk shelf operating in single-bus mode.  Note:  One additional SCSI port and an H8253-AA UltraSCSI bus splitter kit are required to operate StorageWorks disk shelf in split-bus mode.

Note:
See Step 12 for M-series Cabinet Enclosure configuration rules

Pedestal Systems include

· Alpha microprocessor 21264 500 MHz CPU with 4 MB ECC onboard cache

· Pedestal enclosure with:

· 6 expansion slots
Five 64-bit PCI slots and one 64-bit PCI/ISA combination slot

· 16 DIMM memory slots support 4 memory options

· 10 storage slots
One diskette drive slot (1.44MB diskette drive included)
One 600 MB 32X CD-ROM drive slot (CDROM included)
One additional removable media slot
Seven StorageWorks hard drive slots

· Two serial ports, support full duplex asynchronous modem control

· One bi-directional enhanced parallel port

· PS/2 style keyboard port and mouse port

· One 675-Watt self-switching (120/240V, 60/50Hz) power supply1
· 1.44 MB diskette drive in dedicated slot

· 600 MB CD-ROM—uses one removable media slot

· 128MB,  256MB, or 512 MB of memory

· 3-button mouse

· U.S. English keyboard and 120 V power cord with North American orders.

· Selection of a country-specific keyboard and power cord is mandatory for all non-North American orders.

· Integrated Advanced Server Management features, including ServerWorks Manager kit

· Hardware Warranty: Three-year on-site2
· Software Warranty2
1.
Included power supply provides power for normal system operation.  System supports an additional optional power supply for N+1 redundancy.

2.
Service upgrades are available; see Hardware and Software Supplemental Services.

Windows NT 6/500 MHz CPU Packaged Systemsxe "DN-55NJA"
· Compaq’s OEM license agreement for Windows NT mandates compliance with specific configuration rules:

· Selection of language-specific Windows NT Server 4.0 license/media (CD-ROM) kit is mandatory, see 
Step 11.

Note:
Windows NT Pedestal Packaged systems have six available PCI slots.  Installation of mandatory options and reduces available PCI slots.
	Order Number
	Memory
	2 SCSI Ports
	Hard Disk Drive
	Network Adapter
	Graphics1
	Monitor

	DN-55NJA-BA
	128 MB
	Mandatory
	Mandatory
	Mandatory
	Mandatory
	Required

	DN-55NJA-CA
	256 MB
	Mandatory
	Mandatory
	Mandatory
	Mandatory
	Required

	DN-55NJA-EA
	512 MB
	Mandatory
	Mandatory
	Mandatory
	Mandatory
	Required


Key:
Mandatory items must be on purchase order at initial order acceptance.


Required items are essential for full system operation.

Step 1b—Mandatory Options

· Selection of a minimum of two SCSI ports, a system disk drive, and a Network Adapter is mandatory.

· Additional options are supported, select as required.

Select one System Disk Drive—Mandatory

If more than one size of disk is ordered, operating system will be factory installed on the smallest disk.

	Order Number
	System Disk 

	DS-RZ1CF-VW
	4.3 GB 7200 RPM UltraSCSI disk

	DS-RZ1DF-VW
	9.1 GB 7200 RPM UltraSCSI disk

	DS-RZ1EF-VW1
	18.2 GB 7200 RPM UltraSCSI disk


1.
18.2GB disk drives are not supported on KZPAC-xx PCI-based RAID controllers

Select two SCSI ports(Mandatory

· Two SCSI ports are mandatory to support internal storage:

· Internal removable media devices are supported on KZPCM-DA and KZPBA-CA adapters only.
Internal removable media devices are not support on KZPAC-AA/CA/CB RAID controllers.

· Internal StorageWorks seven disk shelf requires one SCSI port when configured in single-bus mode1
· System includes SCSI cable(s) to connect SCSI adapters/controllers to internal storage.

1.
To configure StorageWorks disk shelf in split-bus mode one addition SCSI port is required.

The requirement of two SCSI ports can be fulfilled by ordering:

· One KZPCM-DA two port UltraSCSI single-ended adapter, or

· Two KZPBA-CA one port UltraSCSI single-ended adapter, or

· One KZPBA-CA one port UltraSCSI single-ended adapter and one KZPAC-AA/CA/CB UltraSCSI RAID controller.

	Order Number
	SCSI Adapter

	KZPCM-DA
	PCI-based two port UltraSCSI Single-ended adapter and 10/100 Mbit Ethernet Combo adapter 

	KZPBA-CA
	PCI-based one port UltraSCSI Single-ended adapter

	KZPAC-AA
	PCI-based one port UltraSCSI RAID controller

	KZPAC-CA/CB
	PCI-based three port UltraSCSI RAID controller


Select one Ethernet Adapter(Mandatory

Note:
Ethernet adapter is not required if KZPCM is selected as the SCSI adapter.

	Order Number
	Network Adapter

	DE500-BA
	PCI-based 10/100 Mbit Fast Ethernet adapter UTP

	DE504-BA
	PCI-based 10/100 Mbit Quad Channel Ethernet adapter


Step 2—Additional CPU Modules (SMP Add-ons and Upgrades)

AlphaServer DS20 System supports one additional CPU

	3X-KN310-AD
	Windows NT SMP upgrade includes one 6/500 CPU


Step 3—Memory 

· System supports up to four memory kits (each kit includes 4 industry-standard ECC DIMMs).

· System includes one memory kit.

· System supports three additional memory kits for a system maximum of 2 GB memory.

	MS340-BA
	128 MB
(4 x 32 MB DIMMs) memory kit

	MS340-CA
	256 MB
(4 x 64 MB DIMMs) memory kit

	MS340-DA
	512 MB
(4 x 128 MB DIMMs) memory kit


Step 4—Internal Storage

Pedestal Enclosure supports a total of ten storage devices:

· One dedicated diskette drive slot (1.44 MB diskette drive included).

· One 5.25” removable media slot (600 MB 32X CD-ROM included).

· One 5.25” additional removable media slot.

· Seven 3.5-inch internal hard disk drive slots.

Step 4a—Internal Disk Drives

· Selection of one System Disk Drive is mandatory.

· Up to six additional hard disk drives are supported in internal storage disk shelf.

Note:
Internal storage disk shelf is configured in single-bus mode.  To configure internal storage disk shelf in split-bus mode one additional SCSI port and an H8253-AA UltraSCSI bus splitter kit are required.

	DS-RZ1CF-VW
	4.3 GB 7200 RPM 16-bit UltraSCSI disk drive

	DS-RZ1DF-VW1
	9.0 GB 7200 RPM 16-bit UltraSCSI disk drive

	DS-RZ1EF-VW2
	18.0 GB 7200 RPM 16-bit UltraSCSI disk drive


1.
KZPAC RAID Array 230/Plus Controller supports 9 GB Drives – for configuration information on the 32GB Logical Drive Limit of the KZPAC, see the Systems and Option Catalog file for Storage RAID Controllers http://webir.das.dec.com/info/CU5911/CU5911HM.HTM 
2.
18.2 GB disk drives are not supported on KZPAC-xx RAID controllers.

Internal UltraSCSI Cable Kit

	H8253-AA
	UltraSCSI bus splitter kit includes SCSI bus repeater card, two data cables, Y power cable harness, and SCSI active terminator assembly, and installation guide.  Required to configure internal storage shelf in split bus mode.


Internal Removable Media Devices

· Systems include CD-ROM drive in 5.25” half-height removable media slot.

· Systems support one additional 5.25” half-height removable media device.

	RRD47-VD
	600 MB 32X CD-ROM drive (RRD47) 5.25” Top Gun Blue

	TLZ10-LK
	12/24 GB  4 mm DAT SCSI tape drive (TLZ10) 5.25” Frost White


Step 5(External Storage

UltraSCSI Configuration Rules

· UltraSCSI configurations require UltraSCSI components (controllers, adapters, shelves, disks, and cables).

· UltraSCSI adapters and RAID controllers support UltraSCSI disks at UltraSCSI speeds in UltraSCSI Top Gun Blue shelves (DS-BA356-xx).

· Fast10 BA356-xx SCSI shelves can not be retrofitted.  Replace Fast10 SCSI shelf with DS-BA356-xx shelf when adding an UltraSCSI disk and when connecting to UltraSCSI adapter.

· UltraSCSI and Fast10 drives can be mixed in DS-BA356 Top Gun Blue UltraSCSI shelf.  UltraSCSI adapter negotiates maximum transfer speed of drives.

UltraSCSI Connectors and Cables

· KZPBA-CA PCI one port UltraSCSI Single-ended adapter, 68HD connector, requires BN38C cables (2 to 20 m) to connect to UltraSCSI shelf.

· KZPBA-CB PCI one port UltraSCSI Differential adapter, 68HD connector, requires BN38C cables (2 to 25 m) to connect to UltraSCSI shelf or external RAID array controller.

· KZPAC-AA/CA/CB RAID controller, VHDCI connector, require BN37A cables (.3 to 20 m) to connect to UltraSCSI shelf.

· KZPCM-DA PCI two port UltraSCSI Single-ended adapter and 10/100M-bit Ethernet Combo adapter.  Internal 68-pin and 50-pin HD connectors and two external 68-pin VHDCI connectors.

BN38C VHDCI to 68HD Cables(KZPBA UltraSCSI adapter to UltraSCSI shelf

	BN38C-xx
	VHDCI male to 68HD male.  xx=2, 3, 5, 10, and 20 meter lengths


BN37A VHDCI to VHDCI Cables(KZPAC RAID controller and KZPCM UltraSCSI adapter to UltraSCSI shelf

	BN37A-0C
	0.3 m VHDCI male to VHDCI male

	BN37A-0E
	0.5 m VHDCI male to VHDCI male

	BN37A-01
	1.0 m VHDCI male to VHDCI male

	BN37A-1E
	1.5 m VHDCI male to VHDCI male

	BN37A-02
	2.0 m VHDCI male to VHDCI male

	BN37A-2E
	2.5 m VHDCI male to VHDCI male

	BN37A-xx
	3.0 m VHDCI male to VHDCI male.  xx=3, 5, 10, 15, 20, and 25 meter lengths


UltraSCSI StorageWorks Pedestals and Shelves

UltraSCSI Top Gun Blue shelf provides support for UltraSCSI disk drives.  Shelf is available in single-bus or split-bus configurations.  Shelf includes a personality module and a 180 W power supply.

UltraSCSI shelf supports:

· Seven 3.5-inch hard disk drives, or

· One 5.25-inch device and four 3.5-inch hard disk drives, or

· Two 5.25-inch devices and one 3.5-inch hard disk drive.

Note:
Installation of redundant power supply reduces number of 3.5” drive slots by one.

Pedestal Enclosure

	DS-BA356-KF
	UltraSCSI single-ended pedestal, single-bus personality module, 180 W power supply

	DS-BA356-KG
	UltraSCSI single-ended pedestal, split-bus personality module, 180 W power supply

	DS-BA356-KH

	UltraSCSI pedestal, differential personality module, 180 W power supply

	DS-BA35X-HH
	180 W power supply


Tabletop Tape Drives

	DS-TLZ10-DB1
	12/24 GB 4 mm DAT narrow single-ended tabletop SCSI tape, includes 120 V North American power cord, requires BN31W-xx SCSI cable


1.
Requires dedicated KZPBA-CA UltraSCSI adapter, one BN21K-xx SCSI cable, and a country-specific power cord.

Step 5a(Storage Controllers

· Selection of two SCSI ports is mandatory for internal storage, see Step 1c.

· Charts indicate minimum level of operating system supported.

	
Order Number
	
Description
	Maximum # Supported
Windows NT 4.0 SP3

	KZPCM-DA
	PCI-based two port UltraSCSI Single-ended adapter and 10/100Mbit Ethernet Combo adapter, uses 1 PCI slot.  Internal 68-pin and 50-pin HD connectors and two external 68-pin VHDCI SCSI connectors.
	4

	BN37A-xx
	VHDCI male to VHDCI male UltraSCSI cable; required for each external UltraSCSI shelf attached to KZPCM.
	


	KZPBA-CA
	PCI-based one port UltraSCSI Single-ended adapter, uses 1 PCI slot.  Internal 68-pin and 50-pin HD connectors and external 68-pin HD connector.
	4

	KZPBA-CB
	PCI-based one port UltraSCSI Differential adapter, uses 1 PCI slot, external 68-pin HD connector.
	4

	BN38C-xx
	68-pin HD male to VHDCI male UltraSCSI cable; required for each external UltraSCSI shelf attached to KZPBA-CA/CB adapter.
	


	KZPAC-AA1
	PCI-based one port UltraSCSI RAID controller with 4 MB cache memory, includes RAID Array 230/Plus Subsystem software and documentation kit.  VHDCI female connector.  Tape drives are not supported.
	2

	KZPAC-CA1
	PCI-based three port UltraSCSI RAID controller with 4 MB cache memory, includes RAID Array 230/Plus Subsystem Software and documentation kit.  Tape drives are not supported.  SCSI cable is required for each active port.  Requires BN31K-0E for external third port connection.
	2

	KZPAC-CB1
	Same as KZPAC-CA with 8 MB cache memory
	2

	KZPSC-UB
	Cache battery backup for KZPAC controller
	

	BN37A-xx
	VHDCI male to VHDCI male UltraSCSI cable; required for each external UltraSCSI StorageWorks shelf attached to KZPAC.  Note: Manufacturing may substitute BN37A-02 cable for certain cabinet configurations
	

	BN31K-0E1
	SCSI cable/bulkhead assembly kit with KZPAC-CA/CB, allows external connection of one third port.
	

	KZPAC-SB1
	SCSI cable/bulkhead assembly kit with two ports for KZPAC-CA/CB, allows external connection of two third ports.
	


1.
18.2 GB disk drives are not supported on KZPAC-xx RAID controllers

2.
SCSI cable/bulkhead assembly kit supports one third port connection from KZPAC-CA/CB, allows external connection of one third port.  AlphaServer DS20 includes ten bulkhead slots, six are active PCI I/O slots.

3.
SCSI cable/bulkhead assembly kit supports two third port connections from KZPAC-CA/CB, allows external connection of two third ports.

[image: image2.wmf]
Step 5b(M-series Cabinet Enclosure

For additional information on M-series Cabinet Enclosures see www.digital.com/alphaserver/products/m-series.html 

M-series Top Gun Blue Cabinet Enclosures are available in two sizes:

· 67” high Cabinet Enclosure (H9A10-xx) 31U or 54.2” of usable vertical space.

· 79” high Cabinet Enclosure (H9A15-xx) 38U or 66.5” of usable vertical space.

· AlphaServer DS20 Pedestal requires 11U of usable vertical space.

· StorageWorks shelves require 4U of usable vertical space.

Cabinet Configuration Chart

Note:
Order a Mounting Kit for each system installed in a Cabinet Enclosure.

	Cabinet Enclosure
	Mounting Kit
	Cabinet Height and Space
	Systems
	Shelves
	Ballast Kit

	H9A10-MC/MD
	2T-R1200-AB
	54.2 inches
	31U
	1
2
	6
2
	1
1

	H9A15-MC/MD
	2T-R1200-AB 
	66.5 inches
	38U
	1
2
3
	6
6
2
	2
1
0


M-series Cabinet Enclosure Options

M-series Cabinet Enclosures require the mandatory selection of:

· H9A10 or H9A15 Cabinet, 120V or 240V variant.

· AlphaServer DS20 Rackmount Kit per system ordered.

· One 4.5 meter power cord per system ordered.

· Cabinet front door based on cabinet height.

· Factory Integration.

· Ballast kits as required.

Cabinet Enclosures

	H9A10-MC
	120 V dual 24 A controller, each with 10 NEMA 5-15R outlets, input power plug NEMA L5-30P, Americas, Asia Pacific
Includes rear door, top fan tray, and 2 H7600-AA power distribution units

	H9A10-MD
	240 V dual 16 A controller, each with 12 IEC C13 outlets, input power plug IEC 309, 16A, Europe
Includes rear door, top fan tray, and 2 H7600-DB power distribution units


	H9A15-MC
	120 V dual 24 A controller, each with 10 NEMA 5-15R outlets, input power plug NEMA L5-30P, Americas, Asia Pacific
Includes rear door, top fan tray, and 2 H7600-AA power distribution units

	H9A15-MD
	240 V dual 16 A controller, each with 12 IEC C13 outlets, input power plug IEC 309, 16A, Europe
Includes rear door, top fan tray, and 2 H7600-DB power distribution units


For custom integration of systems, options, and additional Compaq and third party devices, or for other cabinets, doors, and trim kits of a different color, contact Compaq CustomSystems at 1-800-DIGITAL, or contact your local authorized Compaq Certified Integration Partner.

Rackmount Kit

	2T-R1200-AB
	Rackmount Kit includes mounting shelf, slide rails, front dress bezel, interlock mechanism, and cable management bracket.  System slides out front of cabinet for full service access. Each System installed in a cabinet requires one 4.5 meter power cord for each power supply ordered with system.  BN20Z-4E for North American use, or BN35S-4E for non-North American use.

	BN20Z-4E
	4.5 meter power cord for North American use

	BN35S-4E
	4.5 meter power cord for non North American use


Cabinet Front Door

	H9C10-TF
	Top Gun Blue front door for H9A10-MC/MD Cabinet Enclosure

	H9C15-TF
	Top Gun Blue front door for H9A15-MC/MD Cabinet Enclosure


Step 5b(M-series Cabinet Enclosure (continued)
Factory Integration

	YS-ASCAA-AA
	Factory Integration of systems and storage devices assembled and configured in cabinet enclosure in predefined locations


Ballast Kit

Ballast is added to a cabinet at specific locations for specific configurations to meet safety regulations for installation, operation, and service.  It is required in H9A15 cabinets when only one system is installed.  Refer to Cabinet Configuration Chart.

	H9C10-MC
	Two 35-40 lbs. steel plates


Shelves supported in M-series Cabinets

	BA36R-RC
	UltraSCSI single-ended RETMA rack, single-bus personality module, 180 W power supply

	BA36R-RD
	UltraSCSI single-ended RETMA rack, split-bus personality module, 180 W power supply

	BA35R-MR
	Rackmount StorageWorks controller shelf with RETMA mounting kit, and NEMA power for H9A10-MC/H9A15-MC cabinet, North America, Asia Pacific.  Order BN30B-02 for mounting in 
H9A10-MD/H9A15-MD cabinet (Europe).

	CK-BA35X-HH
	180 W power supply


Shelves supported in SW8xx and 5xx Cabinets

	DS-BA356-JD
	UltraSCSI single-ended METRIC rack, single-bus personality module, 180 W power supply

	DS-BA356-JE
	UltraSCSI single-ended METRIC rack, split-bus personality module, 180 W power supply

	DS-BA356-JF
	UltraSCSI single-ended METRIC rack, single-bus, differential personality module, 180 W power supply


Removable Media Devices for External StorageWorks shelves

	DS-TLZ10-VA
	12/24 GB 4 mm DAT narrow SCSI tape drive in StorageWorks carrier

	TLZ9L-VA
	32-64 GB 4 mm DAT SCSI tape drive loader in full-height StorageWorks carrier

	DS-RRD47-VA
	600 MB 32X CD-ROM drive in StorageWorks carrier

	DS-RRD47-VU
	600 MB 32X CD-ROM, second drive supported in DS-RRD47-VA carrier


StorageWorks Power Supply

	DS-BA35X-HH
	180 W power supply


Compaq CustomSystems Support

Cabinets containing systems and storage must be assembled by qualified assemblers: Compaq Manufacturing, Compaq CustomSystems Group, or an authorized Compaq Certified Integration Partner.

For custom integration of systems, options, cabinet expansion offerings, and additional Compaq and third party devices, or for other cabinets, doors, and trim kits of a different color, contact Compaq CustomSystems at 1 800-DIGITAL, or your local authorized Compaq Certified Integration Partner.

Step 5d(UltraSCSI RAID Array Controller and UltraSCSI RAID Packaged Solutions

UltraSCSI RAID Array Controller

	DS-HSZ70-AH
	StorageWorks UltraSCSI RAID Array controller includes 64 MB cache, 6 UltraSCSI single-ended channels, CLI cable kit, controller to controller jumper cable, and two ECB cables.  Requires HSZ70 Solutions Software Kit and external cache battery (QB-5SBAD-SA/SB for Windows NT).


Step 5d(UltraSCSI RAID Array Controller and UltraSCSI RAID Packaged Solutions (continued)
ESA 10000 Storage Arrays and RAID Array 7000 (RA7000) Options

· ESA 10000 Storage Arrays and RAID Array 7000 (HSZ70 Product Set) are supported on AlphaServer DS20 systems 

· The HSZ70 is supported on KZPBA-CB UltraSCSI differential controller.

ESA 10000 Enterprise Storage Arrays

See StorageWorks Packaged Solutions in StorageWorks Chapter of Systems and Options Catalog, or on the WEB at http://www.digital.com/info/SOHOME/ for additional configuration information on HSZ70 controllers in 
ESA 10000 and RA7000.

	DS-SWXES-AA/AB
	ESA 10000 high capacity/general business base unit

	
	Includes: Data Center 600 mm enclosure
2 BA370-AA Rackmounted shelves with 5 shelf power supplies each, expandable to 8
2 HSZ70 six port controllers 64 MB mirrored write back cache each, expandable to 128 MB each
6 Ultra SCSI expansion cables
1 10 meter host to controller cable (BN37A-10)
Serial line assembly with adapters (9 pin and 25 pin)
Power cord and documentation
Supports up to 48 drives

	
	Requires:
HSZ70 Solutions Software kit for platform, host adapter, and disks to be 

ordered separately
Options:
64 MB cache upgrade and fully redundant power


	DS-SWXES-BA/BB
	ESA 10000 high bandwidth base unit

	
	Includes:
Data Center 600 mm enclosure
2 BA370-AA Rackmounted shelves with 5 shelf power supplies each, expandable to 8
4 HSZ70 six port controllers 64 MB mirrored write back cache each, expandable to 128 MB each
2 10 meter host to controller cables (BN37A-10)
Serial line assembly with adapters (9 pin and 25 pin)
Power cord and documentation
Supports up to 48 drives

	
	Requires:
HSZ70 Solutions Software kit for platform, host adapter, and disks to be 

ordered separately
Options:
64 MB cache upgrade and fully redundant power


	DS-SWXES-CA/CB
	ESA 10000 dual expansion base unit

	
	Includes:
Data Center 600 mm enclosure
2 BA370-AA Rackmounted shelves with 5 shelf power supplies each, expandable to 8
12 Ultra SCSI expansion cables
SW600 cabinet joiner kit
Power cord and documentation
Supports up to 48 drives

	
	Requires:
HSZ70 Solutions Software kit for platform, host adapter, and disks to be 

ordered separately
Options:
Fully redundant power


	DS-SWXES-DA/DB
	ESA 10000 single expansion w/ dual controllers base unit

	
	Includes:
Data Center 600 mm enclosure
1 BA370-AA Rackmounted shelf with 5 shelf power supplies, expandable to 8
2 HSZ70 six port controllers 64 MB mirrored write back cache each, expandable to 128 MB each
1 10 meter host to controller cable (BN37A-10)
Serial line assembly with adapters (9 pin and 25 pin)
Power cord and documentation
Supports 24 drives, expandable to 48 with BA370 rackmount upgrade.

	
	Requires:
HSZ70 Solutions Software kit for platform, host adapter, and disks to be 

ordered separately
Options:
64 MB cache upgrade, fully redundant power, and BA370 rackmount upgrade.


Step 5d(UltraSCSI RAID Array Controller and UltraSCSI RAID Packaged Solutions (continued)
RAID Array 7000 (RA7000)

	DS-SWXRA-HA
	RAID Array 7000 with Dual controllers

	
	Includes: 24 SBB Departmental Cabinet
2 HSZ70 6 port controllers with 64 MB mirrored write-back cache each, expandable to 128 MB each
I/O expansion module
Dual cache battery in SBB with cable
Five 180 watt power supplies, expandable to eight
Fully redundant cooling
Environmental Monitor Unit (EMU)
5 meter host to controller cable BN37A-05 with BN38E-0B VHDCI to 68 HD conversion cable, 
Serial line kit, controller to controller jumper cable for redundant controllers, and U.S. power cord.

	
	Requires:
HSZ70 Solutions Software Kit for platform, host adapter, and disks to be 

ordered separately
Options:
64 MB cache upgrade, Up to two RA7000 Expansion cabinets. Optional power supplies.


	DS-SWXRA-HB
	RAID 24 UltraSCSI SBB Pedestal Expansion cabinet

	DS-SWXRA-HC
	RAID Array 7000 with Single controller

	
	Includes: 24 SBB Storage Cabinet
1 HSZ70 6 port controller with 64 MB mirrored write-back cache, expandable to 128MB
I/O expansion module
Single cache battery in SBB with cable
Five 180 watt power supplies expandable to eight
Fully redundant cooling
Environmental Monitor Unit (EMU)
5 meter host to controller cable BN37A-05 with BN38E-0B VHDCI to 68 HD conversion cable, 
Serial line kit, controller to controller jumper cable for redundant controllers, and U.S. power cord.

	
	Requires:
HSZ70 Solutions Software Kit for platform, host adapter, and disks to be 

ordered separately
Options:
Second HSZ70 controller and cache battery; 64 MB cache upgrade; Up to two RA7000 

Expansion cabinets. Optional power supplies.


Adapters and Platform Specific Solutions Software

· Each KZPBA-CA adapter ordered to support HSZ70(s) requires an HSZ70 Solutions Software Kit

· HSZ70 Solutions Software Kits must be ordered for each new HSZ70 controller in a configuration.

· HSZ70 Solutions Software Kits provide tools needed to link ESA 10000 to a wide variety of vendor platforms.

· HSZ70 Solutions Software Kits with -SA variants included documentation
HSZ70 Solutions Software Kits with -SB variants do not include documentation, select for each additional adapter ordered if documentation is available onsite.

HSZ70 Solutions Software Kits include:

· PCMCIA card containing software for storage controller

· StorageWorks Command Console (SWCC) software, and software licenses

· HSZ70 and SWCC supporting documentation

Select Adapter and HSZ70 Solutions Software Kit for host platform

	Supported Adapters
	HSZ70 Solutions Software Kit
	Host Platform

	KZPBA-CB
	QB-5SBAD-SA/SB
	Windows NT


Cache Upgrade

Select cache upgrade for HSZ70 controllers.  Redundant controllers require equal amounts of cache

	DS-HSSIM-AB
	64 MB Cache Upgrade for HSZ70


Step 5d(UltraSCSI RAID Array Controller and UltraSCSI RAID Packaged Solutions (continued)
Disk Expansion Cabinet

Note:
Order Expansion Cable Kit for each Disk Expansion Cabinet selected.

	DS-SWXRA-HB
	Disk Expansion Cabinet, includes 24 disk slots, 5 power supplies, redundant cooling, EMU and PVA, power cable

	DS-BNK37-1E
	Expansion Cable Kit, required for each Disk Expansion Cabinet DS-SWXRA-HB


	DS-BA35X-HH
	180 Watt Power Supply.  Fully redundant power requires three additional power supplies, one additional DS-BA35X-HE power control unit, and one additional DS-SW6XP-AA/AB power distribution unit.

	DS-BA35X-HE
	ac Power Control Unit, required when more than five 180 Watt power supplies are installed

	DS-SW6XP-AA/AB
	SW600 Power Distribution Unit, quantity of 1 required for each SW600 for full power redundancy.


Additional Options

	DS-HS35X-BC
	Single replacement external cache battery, one battery in a single Blue SBB

	DS-HS35X-BD
	Dual replacement external cache battery, two batteries in a single Blue SBB, supports cache of dual redundant controllers.  Requires ordering 1 Power Verification and Addressing (PVA) DS-BA35X-EC

	DS-BA35X-BA
	Battery shelf for SW600 cabinet

	DS-BA35X-MK
	Dual speed fan kit

	DS-BA35X-MN
	Single-ended I/O module

	DS-BA35X-EB
	Environmental Monitor Unit (EMU)

	DS-BA35X-EC
	Power Verification and Addressing (PVA)

	BN37A-xx
	Host to controller cable

	BN38E-0B
	68-pin HD to VHDCI UltraSCSI conversion cable

	H9C10-JC
	H9A10 Cabinet Joiner Kit for SW600 

	H8865-AA
	UltraSCSI Single-ended external terminator

	H8863-AA
	UltraSCSI Differential external terminator

	DS-BA370-AA
	Rackmountable BA370 shelf includes five 180 W power supplies, eight high power blowers, RETMA and Metric mounting kit, not supported in M-series Cabinet Enclosure.

	DS-SW600-AA
	60 Hz 600 mm Storage Cabinet includes single phase power distribution unit DS-SW6XP-AA

	DS-SW600-AB
	50 Hz 600 mm Storage Cabinet includes single phase power distribution unit DS-SW6XP-AA


Step 5—Monitors

Windows NT systems require a video monitor and keyboard for user stations unless available on site.

Note:
Selection of a video extension cable and a country specific power cord is mandatory for all monitor variants.
See Step 5a for video extension cable
See Step 5b for country specific power cords

	SN-VRQV5-24/23 
	15" (13.8" viewable image size) Corporate Series auto-scan color monitor, flat square Invar CRT, 
0.28 mm dot pitch, VGA to 1024 x 768 @ 75 Hz, TCO 95, MPRII, Energy Star, attached 1.8 meter video cable.  Requires mandatory selection of video extension cable and country specific power cord for all variants.
–24 = Northern Hemisphere without power cord
–23 = Southern Hemisphere without power cord


	SN-VRQP7-24/23
	17” (16.0” viewable image size) professional series auto-scanning color monitor, Trinitron CRT, 0.25 mm aperture grill pitch, VGA to 1280 x 1024 at 75 Hz, TCO 95, MPR-II, Energy Star, attached 1.8 meter video cable.  Requires mandatory selection of video extension cable and country specific power cord for all variants.
–24 = Northern Hemisphere without power cord
–23 = Southern Hemisphere without power cord


Step 5—Monitors (continued)
	SN-VRQP1-24/23
	21" (19.6" viewable image size) auto-scanning color monitor, Trinitron CRT, 0.25 mm aperture grill pitch, VGA to 1600 x 1200 at 75 Hz NI, TCO 95, Energy Star, includes a 1.8 meter video cable. Requires mandatory selection of video extension cable and country specific power cord for all variants.
–24 = Northern Hemisphere without power cord
–23 = Southern Hemisphere without power cord


Step 5b¾Video Extension Cable

	BN39C-02
	1.8 meter video extension cable—mandatory for each monitor ordered


Step 5c¾Monitor Power Cords

	BN26J-1K
	North American, Japan, 120 V, 75-inches long

	BN19H-2E
	Australia, New Zealand, 2.5m long

	BN19C-2E
	Central Europe, 2.5 m long

	BN19A-2E
	UK, Ireland, 2.5 m long

	BN19E-2E
	Switzerland, 2.5 m long

	BN19K-2E
	Denmark, 2.5 m long

	BN19Z-2E
	Italy, 2.5 m long

	BN19S-2E
	Egypt, India, South Africa, 2.5 m long

	BN18L-2E
	Israel, 2.5 m long


Step 6—Graphics Adapters

· Windows NT systems require a graphics adapter, monitor and keyboard for user stations unless available on site.

· System supports maximum of one graphics adapter.

	SN-PBXGB-AA
	Powerstorm 3D30 Graphics (TGA2)


Step 7Networks and Communications

	
Order Number
	
Description
	Maximum # Supported
Windows NT 4.0 SP3

	DE500-BA
	PCI-based 10/100-Mbit 1 port Fast Ethernet controller (Twisted Pair)
	2

	DE504-BA
	PCI-based 10/100-Mbit 4 Port Fast Ethernet controller (Twisted Pair)
	1

	BN25G-xx
	Cable for 10BaseT Twisted Pair connection
	


	DEFPA-AB
	PCI-based FDDI Adapter, single attachment station (SAS), multi-mode fiber (MMF), SC connector
	2

	DEFPA-DB
	PCI-based FDDI Adapter, DAS, MMF, SC connector
	2

	BN34D-xx
	MIC-SC dual fiber optic cable
	

	BN34B-xx
	SC to SC dual fiber optic cable
	


	DEFPA-UB
	PCI-based FDDI Adapter, SAS, TP-PMD, requires BN25H-03
	2

	DEFPA-MB
	PCI-based FDDI Dual Adapter, DAS, TP-PMD, requires BN25H-03
	2

	BN25H-03
	3 m (9.8 ft.) FDDI crossover cable
	


Step 8—Additional Power Supply

· One additional power supply may be added to AlphaServer DS20 Pedestal system for N+1 redundancy.

· Redundant Power Supply requires a country-specific power cord.

	H7899-AA
	675 W Redundant Power Supply Option, requires country-specific power cord


Step 9(Keyboards, Power Cords, and Documentation

Keyboards

Systems ordered in North America include U.S. English keyboard and 120V power cord.  Selection of a country-specific keyboard and power cord is mandatory for all non-North American orders.

	Windows NT Keyboard
	Country

	SN-LKQ47-AA
	US/English

	SN-LKQ47-AB
	Belgian

	SN-LKQ47-AC
	Canadian/French

	SN-LKQ47-AD
	Danish

	SN-LKQ47-AE
	UK

	SN-LKQ47-AF
	Finnish

	SN-LKQ47-AG
	German

	SN-LKQ47-AI
	Italian

	SN-LKQ47-AK
	Swiss/French

	SN-LKQ47-AM
	Swedish

	SN-LKQ47-AN
	Norwegian

	SN-LKQ47-AP
	French

	SN-LKQ47-AR
	Latin-American

	SN-LKQ47-AS
	Spanish

	SN-LKQ47-AV
	Portuguese

	SN-LKQ47-AX
	Bhcsy

	SN-LKQ47-BA
	International

	SN-LKQ47-BH
	Greek

	SN-LKQ47-BI
	Taiwanese

	SN-LKQ47-BK
	Korean

	SN-LKQ47-BP
	Polish

	SN-LKQ47-BQ
	Hungarian

	SN-LKQ47-BR
	Arabic

	SN-LKQ47-BT
	Cyrillic

	SN-LKQ47-BU
	Turkish

	SN-LKQ47-BV
	Czech

	SN-LKQ47-CB
	Thai

	SN-LKQ47-CZ
	Slovak


Pedestal Power Cords

Systems ordered in North America include one BN26J-1K 120 V North American power cord.  Order a country-specific power cord for all systems ordered in Europe and AP.

	BN26J-1K
	North American, Japan, 120 V, 75-inches long

	BN19H-2E
	Australia, New Zealand, 2.5m long

	BN19C-2E
	Central Europe, 2.5 m long

	BN19A-2E
	UK, Ireland, 2.5 m long

	BN19E-2E
	Switzerland, 2.5 m long

	BN19K-2E
	Denmark, 2.5 m long

	BN19Z-2E
	Italy, 2.5 m long

	BN19S-2E
	Egypt, India, South Africa, 2.5 m long

	BN18L-2E
	Israel, 2.5 m long


Step 9(Keyboards, Power Cords, and Documentation (continued)
Rackmount Power Cords

Each system installed in a cabinet requires a Rackmount Kit 2T-R1200-AB, and one 4.5 meter power cord for each power supply ordered with system.

	BN20Z-4E
	4.5 m rackmount power cord for North America

	BN35S-4E
	4.5 m rackmount power cord for non-North America


Documentation

	QZ-014AA-GZ
	Additional AlphaServer DS20 User Guide (print and CD), Installation Card, and Release Notes 


Step 10—Printers and Remote Console

Systems include two full duplex asynchronous serial ports and one bi-directional enhanced parallel port.

Asynchronous serial ports have 9-pin D-subminiature connectors. A 9-pin to MMJ adapter (H8571-J) is required for each connection.  See printer section of Systems and Options Catalog for serial and parallel printer information.

Step 11—Software

· Selection of language specific Windows NT Server license, media kit is mandatory 

· Windows NT Server kits are orderable with packaged systems only.

Windows NT Server plus 10-client access license, media (CD-ROM) kits

	QB-23CAA-SE
	Windows NT Server license, media kit North American English

	QB-23C8A-SE
	Windows NT Server license, media kit International English

	QB-23CPA-SE
	Windows NT Server license, media kit French

	QB-23CGA-SE
	Windows NT Server license, media kit German

	QB-23CSA-SE
	Windows NT Server license, media kit Spanish

	QB-23CUA-SE
	Windows NT Server license, media kit Italian

	QB-23CJA-SE
	Windows NT Server license, media kit Japanese

	QB-23C5A-SB
	Windows NT Server license, media kit Thai

	QB-23CMA-SE
	Windows NT Server license, media kit Swedish

	QB-23CTA-SE
	Windows NT Server license, media kit Hebrew

	QB-23CQA-SE
	Windows NT Server license, media kit Arabic

	QB-23CHA-SE
	Windows NT Server license, media kit Dutch

	QB-23CVA-SE
	Windows NT Server license, media kit Brazilian/Portuguese

	QB-23C4A-SE
	Windows NT Server license, media kit Korean

	QB-23C3A-SE
	Windows NT Server license, media kit Taiwanese

	QB-23C2A-SE
	Windows NT Server license, media kit PRC Chinese


Step 12—Hardware and Software Supplemental Support Services

Hardware—Americas and Asia Pacific only

· Systems include three-year hardware warranty, on-site with 5 x 9, 24-hour response time. 

· Option order as part of the Initial System Sale take on the Warranty of the enclosure

· Option added after the Initial System Sale take on the Warranty of the enclosure until the enclosure warranty ends.

· All unique AlphaServer DS20 options have a minimum of 1 Year Return to Compaq Warranty.

· Select optional Hardware Supplemental Support Services if required.

Step 12—Hardware and Software Supplemental Support Services (continued)
Supplemental Service for System with 1 GB of memory or less.

	FM-TC4HR-36
	5 x 9, 4-hour response time

	FM-TC512-36
	5 x 12, 4-hour response time

	FM-TC616-36
	6 x 16, 4-hour response time

	FM-TC724-36
	7 x 24, 4-hour response time

	FM-TCXHW-60
	Years 1-5, next day, Onsite

	FM-TC4HR-60
	Years 1-5, 5x9, 4-hour response time

	FM-TC512-60
	Years 1-5, 5x12, 4-hour response time

	FM-TC616-60
	Years 1-5, 6x16, 4-hour response time

	FM-TC724-60
	Years 1-5, 7x24, 4-hour response time


Supplemental Service for System with more then 1GB of memory.

	FM-TF4HR-36
	5 x 9, 4-hour response time

	FM-TF512-36
	5 x 12, 4-hour response time

	FM-TF616-36
	6 x 16, 4-hour response time

	FM-TF724-36
	7 x 24, 4-hour response time

	FM-TFXHW-60
	Years 1-5, next day, Onsite

	FM-TF4HR-60
	Years 1-5, 5x9, 4-hour response time

	FM-TF512-60
	Years 1-5, 5x12, 4-hour response time

	FM-TF616-60
	Years 1-5, 6x16, 4-hour response time

	FM-TF724-60
	Years 1-5, 7x24, 4-hour response time


Installation Services

	FM-TC12X-IN
	Hardware product installation for AlphaServer DS20

	FM-TCMEM-IN
	Add-on memory installation for AlphaServer DS20

	FM-TCSMP-IN
	Add-on CPU installation for AlphaServer DS20


Software—Americas and Asia Pacific only

· Software service upgrades for Windows NT include advisory and remedial software support for the time period indicated.

	FM-WNT03-12
	12-month Software Supplemental Support for Windows NT AlphaServer DS20 systems

	FM-WNT03-36
	36-month Software Supplemental Support for Windows NT AlphaServer DS20 systems

	FM-WNT03-60
	60-month Software Supplemental Support for Windows NT AlphaServer DS20 systems


Step 12a—Hardware and Software Supplemental Support Services (Europe only)

Europe does not have specific part numbers for Hardware and Software Supplemental Support Services. Prices are quoted based on worldwide service reference pricing, contact MCS Sales in your country for information on Hardware and Software Supplemental Support Services.

AlphaServer DS20 System Diagram Front View XE "AlphaServer DS20 System Diagram" 
[image: image3.wmf]
AlphaServer DS20 System Diagram Side and Back View

[image: image4.wmf]
System Specifications XE "AlphaServer DS20 specifications" 
	Shipping Dimension

	Height
Width
Depth
Weight
	60 cm (23.8 in.)
43 cm (16.9 in)
65 cm (25.6 in)
43 kg (95 lb) typical
71 kg (156 lb) maximum

	Installed Dimensions

	Height
Width
Depth
Weight
	46 cm (18.1 in.)
35.8 cm (14.1 in.)
58.6 cm (23.1 in.)
39 kg (86 lb) typical
51 kg (113 lb) maximum

	Clearances
	Operating
	Service

	Front
Rear
Sides
	75 cm (29.5 in.)
15 cm (6 in.)
None
	75 cm (29.5 in.)
75 cm (29.5 in.)
75 cm (29.5 in.)

	Environmental

	Temperature
	Operating*
Rate of change

Nonoperating
Storage (60 days)
Rate of change
	1040° C (50104° F)
11° C/hr (20° F/hr)
-4066° C (-40151° F)

-4066° C (-40151° F)
20° C/hr (36F/hr)

	Relative humidity
	Operating
Nonoperating
Storage (60 days)
Rate of change
	10–90%
10–95%
10–95%
20%/hr

	Maximum wet bulb temperature
	Operating 
Storage (60 days)
	28° C (82° F) 
46° C (115°F)

	Minimum dew point temperature
	Operating
Storage (60 days)
	2° C (36° F)
Not tested

	Maximum heat dissipation
Dual supply
	Current 
4097 Btu/hr
	

	Air flow and quality
	Intake location
Exhaust location
Particle size
Concentration
	Front
Rear
N/A
N/A

	Altitude
	Operating†
Nonoperating
	3048 m (10,000 ft)
12192 m (40,000 ft)

	Mechanical shock
	Operating
Nonoperating
	10 G 10 ms
20 G peak 30 ms

	Vibration


	Operating

Nonoperating
	10-500 Hz .1 G peak

1.03Grms  5-300Hz

	Acoustics
	
Operating
Idle
	Average
Declared
5.7 LwA, B  38 LpAm, dba
5.6 LwA, B  37 LpAm, dba

	Electrical—Power Supplies are universal, PFC, auto Ranging, 100/240 Vac 

	Nominal ac voltage
Operating Voltage range
Power source phase
Nominal frequency
Frequency range
Maximum inrush current per line core
RMS current (maximum)
 Single power supply (Single cord set)
 Dual power supply (Multiple cord set)
RMS current (typical)
	100-120 Vac
90-128 Vac
Single
60 Hz
59-61 Hz
11 Amps
12 Amps
7.4 Amps each supply
11.7 Amps @ 100Vac
9.3 Amps @ 120Vac
	220-240 Vac
180-256 Vac
Single
50 Hz
49-51 Hz
22 Amps
5 Amps
3.7 Amps each supply
4.9 Amps @ 220Vac
4.4 Amps @ 240Vac

	Power cord
	Type
Length
U.S. plug
	IEC 320 C13
190 cm (75 in.)
NEMA 5-15


System Specifications (continued)
	Regulatory

	Agency approvals
	UL: Listed to UL1950 (3rd edition)
CSA: Certified to CAN/CSA-C22.2 No. 950-M95
TUV: EN 60950  VDE 0805  GS marked
FCC: Part 15.B Class A

CE:  EN55022, EN50082

VCCI Class II ITE

BCIQ: CISPR22, CNS13438

C-Tick: CISPR22, AS/NZS 3548

	Reviewed to
	AS/NZ 3260:1993 Australian/New Zealand Standard
EN 60950/A4: 1997 European Norm
IEC 950 (2nd edition, 3rd amend)


*
Maximum operating temperature at Sea Level. Reduce by 1 C (1.8 F) for each 600 m (2000 ft) above Sea Level.

†
Higher altitudes are possible if maximum operating temperature is reduced (see Temperature, above); other restrictions may apply, such as maximum permissible altitude for hard drives.

2.2
 Alpha Servers

Alpha Servers 2.1

_977841905.wmf

