AlphaServer ES40 Pedestal(Rackmount



[image: image1.png]



AlphaServer ES40 “Linux-Ready”
V1.0—30 June 1999
Systems and Options Catalog
Product Description 
The Compaq AlphaServer ES40 is a 500 MHz EV6 based Alpha microprocessor server with 4 MB secondary cache with a choice of enclosures without an operating system to allow the installation of Linux..  A choice of enclosures is available:  Tower enclosure, Pedestal enclosure, and a Rack kit that supports installing the system in an M-series cabinet.

Compaq AlphaServer ES40 “Linux-Ready” systems are available in 1 Model

· AlphaServer ES40 Model 1 supports six 64-bit PCI I/O expansion slots and four memory options.

The Model 1 Linux-Ready system supports four CPUs and up to 2GB of main memory, and come standard with a 1.44 MB diskette drive, a CD-ROM drive, two EIA-232 full duplex asynchronous modem control serial ports, one bi-directional enhanced parallel port, one RMC dedicated serial port, and keyboard and mouse ports.  AlphaServer ES40 systems support a maximum of two internal Disk Drive Cages.  Each disk drive cage includes a one-port Ultra2 SCSI adapter and supports up to four 3.5-inch x 1.6-inch hard disk drives.  

System Ordering Rules
· AlphaServer ES40 systems require the mandatory selection of:  One Disk Drive Cage or one StorageWorks Shelf, a system disk, a network adapter and enclosure kit.

M-series Cabinet Enclosures
· Cabinet enclosures are available in two sizes: 67-inch and 79-inch.

· See Cabinet Configuration Chart for maximum combinations of systems and StorageWorks shelves supported in M-series Cabinet Enclosures.

Compaq and the names of Compaq products referenced herein are either trademarks and/or service marks or registered trademarks and/or service marks of Compaq.

DIGITAL is a Trademark of Compaq Computer Corporation. 

Microsoft, Windows, Windows NT, SQL Server, Office and BackOffice are either trademarks or registered trademarks of Microsoft Corporation.

Intel and Pentium are registered trademarks of Intel Corporation.

Other product and company names mentioned herein may be trademarks and/or service marks of their respective owners.

Step 1(Compaq AlphaServer ES40 Systems

Configuration menus require the mandatory selection of:

· Linux-Ready system from Step 1a, or
· Tower enclosure, Pedestal enclosure, or Rack kit from Step 1c, and

· Selection of one Disk Drive Cage with included UltraSCSI adapter, or one StorageWorks Shelf is mandatory.  
If a StorageWorks shelf is selected an UltraSCSI adapter and SCSI cable are mandatory.

· One system disk drive

· One Ethernet adapter

Compaq AlphaServer ES40 systems include

· Compaq AlphaServer ES40 Model 1
· 1 Alpha microprocessor 21264 6/500 MHz CPU with 4 MB cache.

· 6 I/O expansion slots—64-bit PCI 

· Support for 4 memory options (Up to 2GB Max.)
· 1.44 MB diskette drive in dedicated slot

· 600 MB 32X CD-ROM drive in removable media slot

· Two EIA-232 full duplex asynchronous modem control serial ports, 9-pin D-subminiature connectors

· One bi-directional enhanced parallel port, 25-pin 
D-subminiature connector

· One RMC dedicated serial port; 6-pin MMJ connector

· Dual USB ports

· Keyboard and mouse port

· Knockouts for two 68-pin SCSI connections

· One auto sensing power supply

· One 512 MB, 1 GB, or 2 GB memory option—uses one memory option slot (4 DIMMs per memory option).

· U.S. English keyboard with North America orders.

· Selection of a country-specific keyboard is mandatory for all non-North American orders.

· 3-button mouse.

· Integral remote system management console.

· Operating system license

· Customer documentation

· Hardware Warranty1

1.
Service upgrades are available; see Hardware and Software Supplemental Services.

Step 1a(AlphaServer ES40 Linux-Ready Systems

Linux-Ready AlphaServer ES40 Model 1

Linux-Ready AlphaServer ES40 Model 1 systems have six total PCI slots.  Installation of mandatory options 
reduces available slots.


Order Number

CPU and Cache

Memory
I/O
Slots
Supported
Memory Options

Enclosure
Drive Cage
or Shelf
System
Disk Drive
Network Adapter

DJ-62AAA-CA
One 6/500 4 MB
512 MB
6
4
Mandatory
Mandatory
Mandatory
Mandatory

Step 1b(Mandatory Options

· Tower enclosure, Pedestal enclosure, or Rack kit.

· One Disk Drive Cage with included UltraSCSI 
adapter, or
One StorageWorks Shelf
Note:  If StorageWorks shelf is selected an UltraSCSI adapter and SCSI cable are mandatory)

· One system disk drive.

· One Ethernet adapter.

Enclosures and Rack Kit

Selection of a Tower enclosure, Pedestal enclosure, or Rack Kit is mandatory.

Tower Enclosure

· Tower enclosure supports one system.

· Tower enclosure ordered in North America includes a 120 V power cord.  Selection of a country-specific power cord is mandatory for all non-North American orders.

· Ordering an additional power cord is mandatory for each power supply ordered with a system installed in a Tower enclosure.

BA61M-MT
Tower enclosure for AlphaServer ES40

Pedestal Enclosure

· Pedestal enclosure supports one system and up to three UltraSCSI StorageWorks shelves.

· Pedestal systems ordered in North America include two 120 V power cords.  Selection of two country-specific power cords is mandatory for all non-North American orders.

· Pedestal (BA61P-Px) enclosure comes standard with two power strips that are factory configured to allow five free outlets. Should the number of BA356-RC/RD power supplies require a sixth outlet, this can be accommodated, but only if the system chassis is limited to two H7906-A9 power supplies.
BA61P-PA
Pedestal enclosure for AlphaServer ES40 120 V

BA61P-PB
Pedestal enclosure for AlphaServer ES40 240 V

Rack Kit

Selection of a Rack Kit is mandatory for each system installed in a cabinet.

BA61R-RM
Rack kit for AlphaServer ES40

Disk Drive Cage or StorageWorks Shelf—Mandatory

· Selection of one Disk Drive Cage with included UltraSCSI adapter, or one StorageWorks Shelf is mandatory.
If a StorageWorks shelf is selected an UltraSCSI adapter and SCSI cable are mandatory.

· Tower enclosure supports up to two Disk Drive Cages only.

· Pedestal enclosure supports up to two Disk Drive Cages and up to three StorageWorks Shelves.

· M-series cabinet enclosure supports up to two Disk Drive Cages per system and up to six StorageWorks Shelves.

Step 1b(Mandatory Options (continued)

Disk Drive Cage

· System supports up to two Disk Drive Cages.

· Each Disk Drive Cage supports four 3.5-inch x 1.6-inch hard disk drives and includes a one-port Ultra2 SCSI single-ended adapter—Ultra2 SCSI adapter uses one system PCI slot.  Included Ultra2 SCSI adapter is the only adapter supported on the BA610-4D Disk Drive Cage.

BA610-4D
Four bay Disk Drive Cage with Ultra2 SCSI adapter and cable

System Disk Drive—Supported in Disk Drive Cage

Selection of one disk drive is mandatory.

DS-RZ2DD-16
9.1 GB 10000 RPM Ultra2 SCSI drive

DS-RZ2ED-16
18.2 GB 10000 RPM Ultra2 SCSI drive

StorageWorks Shelf

Selection of a StorageWorks shelf requires the mandatory selection of an UltraSCSI adapter and SCSI cable, see Step 5a.

BA36R-RC
StorageWorks UltraSCSI single-bus shelf kit

BA36R-RD
StorageWorks UltraSCSI split-bus shelf kit

System Disk Drive—Supported in StorageWorks Shelf 

Selection of one disk drive is mandatory.

DS-RZ1DA-VW
9.1 GB 7200 RPM UltraSCSI disk drive

DS-RZ1EA-VW
18.2 GB 7200 RPM UltraSCSI disk drive

DS-RZ1FB-VW
36.4 GB 7200 RPM UltraSCSI disk drive

DS-RZ1DD-VW
9.1 GB 10,000 RPM UltraSCSI disk drive

DS-RZ1ED-VW
18.2 GB 10,000 RPM UltraSCSI disk drive

Ethernet Adapter(Mandatory

· Selection of one Ethernet adapter is mandatory. 

· Ethernet adapter is not required if KZPCM is selected as a SCSI adapter for StorageWorks shelf.

Order Number
Network Adapter 

DE500-BA
PCI-based 10/100 Mbit Fast Ethernet adapter UTP

DE504-BA
PCI-based 10/100 Mbit Quad Channel Fast Ethernet adapter 

BN25G-xx
Cable for 10BaseT Twisted Pair connection

Step 1c—M-series Cabinet Enclosure

For additional information on M-series Cabinet Enclosures see www.digital.com/alphaserver/products/m-series.html 

M-series Top Gun Blue Cabinet Enclosures are available in two sizes:

· 67” high Cabinet Enclosure (H9A10-xx) 32U or 55.9” of usable vertical space.

· 79” high Cabinet Enclosure (H9A15-xx) 38U or 66.5” of usable vertical space.

Note:
Selection of a Rear Cabinet Extender is mandatory for each H9A10 or H9A15 Cabinet, and
Selection of a Rack Kit is mandatory for each system installed in a cabinet.

Cabinet Configuration Chart for AlphaServer ES40 (8U) with StorageWorks Shelves (4U)

Cabinet Enclosure
Rack Kit
Cabinet Extender
Cabinet Height and Space
Systems
Shelves
Ballast Kit

H9A10-MC/MD
BA61R-RM
H9C10-TE
56 inches
32U
1
2
3
41
6
6
2
0


H9A15-MC/MD
BA61R-RM
H9C15-TE
66.5 inches
38U
1
2
3
4
6
6
63
24
12

1.
Contact Compaq CustomSystems for cabinet configurations with four ES40 systems installed in an H9C10-MC/MD cabinet.

2.
Installing one ES40 system in an H9A15 cabinet requires the mandatory selection of a ballast kit.

3.
At 100V a maximum of two shelves are supported.

4.
At 100V no shelves are supported. 

M-series Cabinet Enclosures

M-series Cabinet Enclosure for AlphaServer ES40 requires the mandatory selection of:

· H9A10 or H9A15 Cabinet, 120V or 240V variant.

· Rack kit for each system ordered.

· H9A10 or H9A15 Rear Cabinet Extender.

· Cabinet front door based on cabinet height.

· Factory Integration.

· Ballast kit if only one system is ordered for H9A15 cabinet.

H9A10-MC
120 V dual 24 A controller, each with 10 NEMA 5-15R outlets, input power plug NEMA L5-30P, Americas, Asia Pacific.  Includes rear door and top fan tray.

H9A10-MD
240 V dual 16 A controller, each with 12 IEC C13 outlets, input power plug IEC 309, 16A, Europe. Includes rear door and top fan tray.

H9A15-MC
120 V dual 24 A controller, each with 10 NEMA 5-15R outlets, input power plug NEMA L5-30P, Americas, Asia Pacific. Includes rear door and top fan tray.

H9A15-MD
240 V dual 16 A controller, each with 12 IEC C13 outlets, input power plug IEC 309, 16A, Europe. Includes rear door and top fan tray.

Rear Cabinet Extender

Selection of a Rear Cabinet Extender is mandatory for each H9A10 or H9A15 Cabinet.

H9C10-TE
Rear Cabinet Extender, Top Gun Blue, for H9A10-MC/MD cabinets

H9C15-TE
Rear Cabinet Extender, Top Gun Blue, for H9A15-MC/MD cabinets

Cabinet Front Door

H9C10-TF
Front door, Top Gun Blue, for H9A10-MC/MD Cabinet Enclosure

H9C15-TF
Front door, Top Gun Blue, for H9A15-MC/MD Cabinet Enclosure

Factory Integration

YS-ASCAA-AA
Factory Integration of systems and storage devices assembled and configured in cabinet enclosure in predefined locations

Step 1c—M-series Cabinet Enclosure (continued)

Ballast Kit

Ballast is added to a cabinet at specific locations for specific configurations to meet safety regulations for installation, operation, and service.  See Cabinet Configuration Chart.

H9C10-MC
Two 35-40 lbs. steel plates

Shelves supported in M-series Cabinets

BA36R-RC
UltraSCSI single-ended RETMA rack, single-bus personality module, 180 W power supply

BA36R-RD
UltraSCSI single-ended RETMA rack, split-bus personality module, 180 W power supply

BA35R-MR
Rackmount StorageWorks controller shelf with RETMA mounting kit, and NEMA power for H9A10-MC/H9A15-MC cabinet, North America, Asia Pacific.  Order BN30B-02 for mounting in 
H9A10-MD/H9A15-MD cabinet (Europe).

CK-BA35X-HH
180 W power supply, includes 120 V North American and 240V IEC power cord

Compaq CustomSystems Support

Cabinets containing systems and storage must be assembled by qualified assemblers: Compaq Manufacturing, Compaq CustomSystems Group, or an authorized Compaq Certified Integration Partner.

For custom integration of systems, options, cabinet expansion offerings, and additional Compaq and third party devices, or for other cabinets, doors, and trim kits of a different color, contact Compaq CustomSystems at 1-800-AT COMPAQ, or your local authorized Compaq Certified Integration Partner.

Step 2(Additional CPU Modules

· AlphaServer ES40 systems include one 6/500 MHz CPU, three additional CPUs are supported.

Note:
Power supply included with system supports two CPUs.  See Step 2a for Additional Power Supply information.

KN610-AA
SMP upgrade includes one 6/500 CPU with 4 MB cache 

Step 2a—Additional Power Supply

· Power supply included with System provides support for two CPUs and one Disk Drive Cage.

· Third and fourth CPUs, and/or second Disk Drive Cage require an additional power supply.

· Each additional power supply ordered with a Tower system requires an additional power cord, see Step 9.

H7906-A9
AlphaServer ES40 power supply

AlphaServer ES40

1st
power supply (included)
Supports up to 2 CPUs and one Disk Drive Cage

2nd
power supply
Supports 3rd and 4th CPU, and second Disk Drive Cage, or
Provides N+1 for up to 2 CPUs and one Disk Drive Cage

3rd
power supply
Provides full N+1 redundancy

For Cabinet system configurations that require a third power distribution unit (H7600-xx), contact Compaq CustomSystems at 1-800-AT-COMPAQ (1-800-282-6672).

Step 3(Memory

· System includes one memory option. Maximum of 2GB supported.

· AlphaServer ES40 Model 1 systems support up to three additional memory options (total of 16 DIMMs)

· AlphaServer ES40 Model 2 systems support up to seven additional memory options (total of 32 DIMMs)

· Memory performance considerations: Interleaved operations reduce the average latency and increase the memory throughput over non-interleaved operations.  With 1 memory option (4 DIMMs) installed, memory interleaving will not occur.  With 2 memory options (8DIMMs) installed, memory read-write operations are two-way interleaved. With 4 memory options (16 DIMMs) installed, memory read-write operations are four-way interleaved, maximizing memory throughput.

MS610-BA
256 MB memory option (4 x 64 MB DIMMs)

MS610-CA
512 MB memory option (4 x 128 MB DIMMs)

MS610-DA
1 GB memory option (4 x 256 MB DIMMs)

Step 4—Internal Storage

System supports a total of four internal removable media devices.

· One dedicated diskette drive slot (1.44 MB diskette drive included).

· One 5.25” removable media slot (600 MB 32X CD-ROM included).

Disk Drive Cage

· System supports up to two Disk Drive Cages.

· Each Disk Drive Cage supports four 3.5-inch x 1.6-inch hard disk drives and includes a one-port Ultra2 SCSI single-ended adapter—Ultra2 SCSI adapter uses one system PCI slot.  Included Ultra2 SCSI adapter is the only adapter supported on the BA610-4D Disk Drive Cage.

BA610-4D
Four bay Disk Drive Cage with Ultra2 SCSI adapter and cable

Disk Drives—Supported in Disk Drive Cage 

DS-RZ2DD-16
9.1 GB 10000 RPM Ultra2 SCSI drive

DS-RZ2ED-16
18.2 GB 10000 RPM Ultra2 SCSI drive

Step 5(External Storage

UltraSCSI Configuration Rules

· UltraSCSI configurations require UltraSCSI components (controllers, adapters, shelves, disks, and cables).

· UltraSCSI adapters and RAID controllers support UltraSCSI disks at UltraSCSI speeds in UltraSCSI Top Gun Blue shelves (DS-BA356-xx).

· UltraSCSI and Fast10 drives can be mixed in DS-BA356 Top Gun Blue UltraSCSI shelf.  UltraSCSI adapter negotiates maximum transfer speed of drives.

UltraSCSI Connectors and Cables

· KZPBA-CA PCI one port UltraSCSI Single-ended adapter, 68HD connector, requires BN38C cable (2 to 20 m) to connect to Single-ended UltraSCSI shelf.

· KZPBA-CB PCI one port UltraSCSI Differential adapter, 68HD connector, requires BN38C cable (2 to 25 m) to connect to Differential UltraSCSI shelf

BN38C VHDCI to 68HD Cables(KZPBA UltraSCSI adapter to UltraSCSI shelf

BN38C-xx
VHDCI male to 68HD male.  xx=2, 3, 5, 10, and 20 meter lengths

Step 5a(Storage Controllers—For External StorageWorks Shelves

Note:
These controllers are not supported on the BA610-4D Disk Drive Cage.

KZPBA-CA
PCI-based one port UltraSCSI Single-ended adapter, uses 1 PCI slot.  Internal 68-pin and 50-pin HD connectors and external 68-pin HD connector.



KZPBA-CB
PCI-based one port UltraSCSI Differential adapter, uses 1 PCI slot, external 68-pin HD connector.



BN38C-xx
68-pin HD male to VHDCI male UltraSCSI cable; required for each external UltraSCSI shelf attached to KZPBA-CA/CB adapter.



Step 6(Monitors

Graphics monitors other than those listed below can be used if compatible with SVGA graphics ordered with a system.

Note:
Selection of a video extension cable and a country specific power cord is mandatory for all monitor variants.
See Step 6a for video extension cable
See Step 6b for country specific power cords

3R-VRQV5-24/11 
15" (13.8" viewable image size) Corporate Series auto-scan color monitor, flat square Invar CRT, 
0.28 mm dot pitch, VGA to 1024 x 768 @ 85 Hz, TCO 95, MPRII, Energy Star, attached 1.8 meter video cable.  Requires mandatory selection of video extension cable and country specific power cord for all variants.
–24 = Northern Hemisphere without power cord
–11 = Southern Hemisphere without power cord

SN-VRQP7-24/23
17” (16.0” viewable image size) professional series auto-scanning color monitor, Trinitron CRT, 0.25 mm aperture grill pitch, VGA to 1280 x 1024 at 75 Hz, TCO 95, MPR-II, Energy Star, attached 1.8 meter video cable.  Requires mandatory selection of video extension cable and country specific power cord for all variants.
–24 = Northern Hemisphere without power cord
–23 = Southern Hemisphere without power cord

SN-VRQP1-24/23
21" (19.6" viewable image size) auto-scanning color monitor, Trinitron CRT, 0.25 mm aperture grill pitch, VGA to 1600 x 1200 at 75 Hz NI, TCO 95, Energy Star, includes a 1.8 meter video cable.  Requires mandatory selection of video extension cable and country specific power cord for all variants.
–24 = Northern Hemisphere without power cord
–23 = Southern Hemisphere without power cord

Step 6a(Video Extension Cable

BN39C-02
1.8 meter video extension cable—mandatory for each monitor ordered

Step 6b(Monitor Power Cords

BN26J-1K
North American, 120 V, 75 inches long

3X-BN46F-02
Japan, 2.5 m long 

BN19H-2E
Australia, New Zealand, 2.5m long

BN19C-2E
Central Europe, 2.5 m long

BN19A-2E
UK, Ireland, 2.5 m long

BN19E-2E
Switzerland, 2.5 m long

BN19K-2E
Denmark, 2.5 m long

BN19M-2E
Italy, 2.5 m long

BN19S-2E
Egypt, India, South Africa, 2.5 m long

BN18L-2E
Israel, 2.5 m long

Step 7(Graphics Adapters

· RAID Configuration Utility is accessible via the console port for Tru64 UNIX and OpenVMS systems.  Optional graphics capability is available.

· Select graphics adapter, monitor, and country specific keyboard for Tru64 UNIX and OpenVMS systems if required.

· System supports maximum of one graphics adapter.

SN-PBXGB-AB
ELSA Gloria Synergy 8MB 2D/3D graphics adapter

Step 8(Networks and Communications


Order Number

Description

DE500-BA
PCI-based 10/100-Mbit 1 port Fast Ethernet controller (Twisted Pair)

DE504-BA
PCI-based 10/100-Mbit 1 port Quad Channel Fast Ethernet controller 

BN25G-xx
Cable for 10BaseT Twisted Pair connection

Step 9(Keyboards, Power Cords, and Documentation

Systems ordered in North America include U.S. English keyboard.  Selection of a country-specific keyboard is mandatory for all non-North American orders.

Tru64 UNIX Keyboard
Country

SN-LKQ47-AA
US/English

SN-LKQ47-AB
Belgian

SN-LKQ47-AC
Canadian/French

SN-LKQ47-AD
Danish

SN-LKQ47-AE
UK

SN-LKQ47-AF
Finnish

SN-LKQ47-AG
German

SN-LKQ47-AH
Dutch

SN-LKQ47-AI
Italian

SN-LKQ47-AK
Swiss/French

SN-LKQ47-AM
Swedish

SN-LKQ47-AN
Norwegian

SN-LKQ47-AP
French

SN-LKQ47-AR
Latin-American

SN-LKQ47-AS
Spanish

SN-LKQ47-AT
Hebrew

SN-LKQ47-AV
Portuguese

SN-LKQ47-AX
BHCSY

SN-LKQ47-BA
International

SN-LKQ47-BH
Greek

SN-LKQ47-BI
Taiwanese

SN-LKQ47-BK
Korean

SN-LKQ47-BP
Polish

SN-LKQ47-BQ
Hungarian

SN-LKQ47-BR
Arabic

SN-LKQ47-BT
Cyrillic

SN-LKQ47-BU
Turkish

SN-LKQ47-BV
Czech

SN-LKQ47-CB
Thai

SN-LKQ47-CV
S. Chinese

SN-LKQ47-CZ
Slovak

Tower Power Cords

· Tower enclosure ordered in North America includes a 120 V power cord.  Selection of a country-specific power cord is mandatory for all non-North American orders.

· Each additional power supply ordered with a system installed in a Tower enclosure requires an additional power cord.

Pedestal Power Cords

· Pedestal systems ordered in North America include two 120 V power cords.  Selection of two country-specific power cords is mandatory for all non-North American orders.

BN26J-1K
North American 120 V, 75 inches long

3X-BN46F-02
Japan, 2.5 m long

BN19H-2E
Australia, New Zealand, 2.5 m long

BN19C-2E
Central Europe, 2.5 m long

BN19A-2E
UK, Ireland, 2.5 m long

BN19E-2E
Switzerland, 2.5 m long

BN19K-2E
Denmark, 2.5 m long

BN19M-2E
Italy, 2.5 m long

BN19S-2E
Egypt, India, South Africa, 2.5 m long

BN18L-2E
Israel, 2.5 m long

Documentation

QZ-01BAA-GZ
Additional AlphaServer ES40 System Information Kit (print and CD), Owners Guide, User Interface Guide, and Release Notes

Step 10(Printers and Remote Console

Systems include two full duplex asynchronous serial ports and one bi-directional enhanced parallel port.

Asynchronous serial ports have 9-pin D-subminiature connectors.  A 9-pin to MMJ adapter (H8571-J) is required for each connection.  See printer section of Systems and Options Catalog for serial and parallel printer information.

Step 10a—Console Terminal

· Console terminal with EIA-232 25-pin DSUB connector required unless otherwise available.

· Shielded console cable is required for console terminal unless available onsite.

VT510-xx
VT510 terminal

Step 11(Software

· Operating system software is not supplied with this system

System Diagram

[image: image2.wmf]
1.
Control panel

2.
CD-ROM drive

3.
Removable media bays

4.
Floppy diskette drive

5.
Storage drive bays

6.
Fans

7.
CPUs

8.
Memory

9.
PCI cards

[image: image3.wmf]
1.

Power supplies

2.
PCI bulkhead

3.
I/O ports

System Diagram

[image: image4.wmf]
Tower Specifications

Dimensions—Tower

Height
Width
Depth
Weight
50.8 cm (20.0 in.)
38.7 cm (15.25 in.)
78.7 cm (31.0 in.)
65.6 kg (144.3 lb) Typical
96.7 kg (212.8 lb) Maximum

Shipping Dimension—Tower

Height
Width
Depth
Weight
82.48 cm (32.2 in.)
60.2 cm (24.0 in.)
101.6 cm (40.0 in.)
78 kg (1720 lb) Typical
110 kg (242 lb) Maximum

Clearances
Operating
Service

Front
Rear
Left side
Right side
75 cm (29.5 in.)
15 cm (6 in.)
None
None
75 cm (29.5 in.)
75 cm (29.5 in.)
75 cm (29.5 in.)
None

Pedestal Specifications

Dimensions—Pedestal

Height
Width
Depth
Weight
78.2 cm (30.8 in.)
50.8 cm (20.0 in.)
80.6 cm (31.75 in.)
127 kg (280 lb) Typical
159 kg (350 lb) Maximum

Shipping Dimension—Pedestal

Height
Width
Depth
Weight
107.7 cm (42.4 in.)
100.3 cm (39.5 in.)
60.7 cm (23.9 in.)
Nominal: 149 kg (328 lb.), Maximum: 185 kg (407 lb.)

Clearances
Operating
Service

Front
Rear
Left side
Right side
75 cm (29.5 in.)
15 cm (6 in.)
None
None
75 cm (29.5 in.)
75 cm (29.5 in.)
None
75 cm (29.5 in.)

Rackmount Specifications

Dimensions—Rackmount

Height
Width
Depth
Weight
 When lifting
 Total added to cabinet
 (brackets, slides, & cables)
35.2 cm (13.87 in.) Fits 14 in. [8U] standard RETMA cabinet
44.7 cm (17.6 in.)
76.5 cm (30.1 in.)

Nominal: 50 kg (110 lb.), Maximum 76 kg (167.2 lb.)
Nominal: 59 kg (130 lb.), Maximum 92 kg (202.4 lb.)

Shipping Dimension—Rackmount

Height
Width
Depth
Weight
73.2 cm (28.8 in.)
60.7 cm (24.0 in.)
101.6 cm (40.0 in.)
Nominal: 72 kg (158 lb.), Maximum: 106 kg (233 lb.)

Clearances
Operating
Service


See requirements of specific cabinet
Minimum 121.9 cm (4 ft.)
76.3 cm (30 in.)
withdrawal on rails

Cabinet Specifications

Cabinet Dimensions
H9A10 M-Series
H9A15 M-Series

Height
Width
Depth
Weight
170 cm (67.0 in)
60 cm (23.6 in) Rack 
110 cm (43.27 in)
Configuration dependent
200 cm (79.0 in)
60 cm (23.6 in) Rack 
90 cm (35.4 in)
450 kg (1000 lb)

Shipping Dimensions
H9A10 M-Series
H9A15 M-Series

Height
Width
Depth
Weight
185.5 cm (73 in.)
91.5 cm (36 in.)
122 cm (48 in.)
Configuration dependent
Maximum payload 1000 lb.
216 cm (85 in.)
91.5 cm (36 in.)
122 cm (48 in.)
Normal: 550 kg (1056 lb)
Maximum: 640 kg (1408 lb.)

ES40 System Specifications

Environmental

Temperature

Operating*
 Tower, Pedestal, Rackmount
Nonoperating
Storage (60 days)
Rate of change

10-35° C (50-95° F)
Not tested)
–40 to –66° C (-40 to –151° F)
11° C/hr (20° F/hr)

Relative humidity
Operating
Nonoperating
Storage (60 days)
Rate of change
20–80%
20–80%
10–95%
20% hr.

Maximum wet bulb temperature
Operating 
Storage (60 days)
28° C (82° F) 
46° C (115°F)

Minimum dew point temperature
Operating
Storage (60 days)
2° C (36° F)
Not tested

Heat dissipation
 Tower and Rack
 Pedestal
 H9A10/H9A15
Nominal
 900 W, 3074 BTU/hr
 1480 W, 5054 BTU/hr
 Configuration dependent
Maximum
 1300 W, 4440 BTU/hr
 2400 W, 8196 BTU/hr
 4800 W, 16392 BTU/hr

Air flow and quality
Intake location
Exhaust location

Particle size
Concentration
Front
Rear-Tower, Pedestal, Rack
Rear/Top-H9A10/H9A15
N/A
N/A

Altitude
Operating†
Nonoperating
3037 m (10,000 ft)
12192 m (40,000 ft)

Acoustics—Declared values per ISO 9296 and ISO 7779

Current values for specific configurations are available from Compaq.  1 B = 10 dBA

Acoustics
 Idle
 Operating
LWAd3 B
6.6
6.6
LpAm5 dBA (bystander position)
48
48

Vibration
Operating
Nonoperating
10-500 Hz .1 G peak
1.03 Grms 5-300 Hz

Electrical—Power Supplies are universal, PFC, auto Ranging, 100/240 Vac 

Nominal voltage (Vac)
Voltage range (Vac)
 (temporary condition)
Power source phase
Nominal frequency (Hz)
Frequency range (Hz)
RMS current 
 (maximum steady state)
 Tower and Rackmount
   Single power cord
   Multiple power cords
   Maximum VA
 Pedestal
   Each power cord
   Maximum VA
 M-Series Cabinet 
   (config-dependent)
   Nominal voltage (VAC)
   Each power cord
100 
90-100

Single
50/60
49-51 / 59-61



11.0 A
6.5 A
1300

12.0 A
1960


100
24 A
120
110-128


Single
50/60
49-51 / 59-61



8.5 A
5.3 A
1270

10.5 A
1900


120
24 A
200-240
180-250

Single
50/60
49-51 / 59-61



5.0 A
3.0 A
1250

6.0 A
1880


220-240
16 A

ES40 System Specifications

Power Cords

System
Quantity
Power Cord Length
Type

Tower
Up to 3
190 cm (75 in.)
IEC 320 C13 to NEMA 5-15 
(N. America) or
IEC 320 C13 to country-specific

Pedestal
2
190 cm (75 in.)
120 V nonremovable NEMA 5-15
(N. America) or
200-240 V IEC 320 C13 to 
country-specific

Rackmount
3
452 cm 14 ft. 10 in.)
IEC 320 C13 to NEMA 5-15 
(N. America) or
IEC 320 C13 to IEC 320 C14 
(other countries)

Cabinet
2
330 cm 10ft. 10 in.)
120 V nonremovable NEMA 
L5-30P or
200-240 V nonremovable IEC 309 

Regulatory

Agency approvals
UL: Listed to UL1950 (3rd edition)
CSA: Certified to CAN/CSA-C22.2 No. 950-M95
TUV: EN 60950/A4: 1997 VDE 0805  GS marked
FCC: Part 15.B Class A
CE:  EN55022, EN50082
VCCI Class II ITE
BCIQ: CISPR22, CNS13438
C-Tick: CISPR22, AS/NZS 3548

Reviewed to
AS/NZ 3260:1993 Australian/New Zealand Standard
EN 60950/A4: 1997 European Norm
IEC 950 (2nd edition, 3rd amend)

*
Maximum operating temperature at Sea Level.  Reduce by 1 C (1.8 F) for each 600 m (2000 ft) above Sea Level.

†
Higher altitudes are possible if maximum operating temperature is reduced (see Temperature, above); other restrictions may apply, such as maximum permissible altitude for hard drives.

Alpha Servers 2.1

_977841905.wmf

