AlphaServer DS10



[image: image1.png]



AlphaServer DS10 
 V1.3(11 June 1999
Systems and Options Catalog
[image: image2.jpg]


Product Description
The AlphaServer DS10 is based on the Alpha 21264 microprocessor 466 MHz CPU with 2 MB ECC cache.  Servers are available in Desktop configurations or can be rackmounted in M-series cabinets with a choice of Tru64 UNIX or OpenVMS operating systems. The AlphaServer DS10 is a low-cost entry level server with enterprise level server features.  The DS10 is ideal for ISP/Internet applications, High Performance Technical Computing, software development and replicated site applications, to name a few.  The DS10 has a 3U/5.25” form factor which makes it ideal for rackmounting into cabinets to conserve valuable floor space.  Integrated on the system motherboard are Dual 10/100 BaseT  ethernet ports, Remote Management Console, floppy diskette controller, two serial ports, one parallel port, and keyboard and mouse interfaces.  A SCSI adapter and a SCSI cable kit are required to support internal SCSI disks and tape.

The AlphaServer DS10 supports five internal storage devices, including a floppy diskette, CD-ROM, and up to three SCSI hard disk drives.  System supports up to 1 GB of memory and 27GB of internal storage with three 9GB SCSI disks.

System Ordering Rules 

AlphaServer DS10 packaged systems do not include a system disk.  Selection of a system disk is mandatory.  Users are required to select a SCSI disk as the system disk at the time of system order.   A PCI SCSI controller and SCSI cable kit are also mandatory selections.
To order a complete system, select (1) a System, (2) a System Disk, and (3) a SCSI adapter and SCSI cable kit are also mandatory.

M-series Cabinet Enclosures 

M-series Cabinet Enclosures are available in three sizes: 43 inch, 67 inch, and 79 inch.  See Cabinet Configuration Chart for maximum system and StorageWorks shelves supported in M-series Cabinet Enclosures.

High Availability Features
The AlphaServer DS10 integrated Remote Management Console (RMC) features significantly enhance the ability to remotely manage and diagnose system conditions.  The RMC allows a remote operator to monitor server voltages, temperatures, and fan, as well as manipulate the server (Power On/Off, reset, halt).  An auxiliary regulator powers the RMC in the main power system that allows systems to be remotely interrogated even if systems are turned off or shut down.  In the event of an alert condition, the RMC can be set up to automatically dial a user definable pager phone or another system to make a remote operator aware of the alert condition

Server WORKS Manager provides advanced server and network management capabilities and is supplied with all AlphaServer systems.  Simple Network Management Protocol (SNMP) enables information to pass from the managed system console for Tru64 UNIX and Open VMS systems.  Using ServerWORKS, the system manager may build and view topological maps of the network.  Detailed server information is viewable, including system, network, storage, and environmental information.  Alarms may be set against critical system parameters and events.  All AlphaServer systems are supplied with management tools to complement ServerWORKS Manager.

Compaq and the names of Compaq products referenced herein are either trademarks and/or service marks or registered trademarks and/or service marks of Compaq.  DIGITAL is a Trademark of Compaq Computer Corporation.   Microsoft, Windows, Windows NT, SQL Server, Office and BackOffice are either trademarks or registered trademarks of Microsoft Corporation.  Intel and Pentium are registered trademarks of Intel Corporation.

Other product and company names mentioned herein may be trademarks and/or service marks of their respective owners.

Step 1(AlphaServer DS10 Desktop and Rackmount Systems
AlphaServer DS10 Desktop and Rackmount Systems include:

· Alpha microprocessor 21264 466 MHz CPU with 2 MB onboard ECC cache

· Onboard dual 10/100 Base T Ethernet ports

· Four PCI expansion slots 

· Three 64-bit PCI slots

· One 32-bit PCI slot

· 4 DIMM memory slots support 2 memory kits

· Five storage bays

· One diskette drive bay with 1.44 MB floppy drive

· One CD-ROM drive bay with 600 MB CD-ROM

· One 5.25-inch x 1.6-inch removable media bay or 3.5-inch x 1-inch hard disk drive bay

· Two 3.5-inch x 1-inch hard disk drive bays

· Two serial ports 

· One serial port with full duplex asynchronous modem control

· One RMC dedicated serial port with full duplex asynchronous modem control

· One bi-directional enhanced parallel port

· PS/2 style keyboard port and mouse port

· 300 W (120/240V, 60/50 Hz) power supply

· 3 button mouse

· US English keyboard (North American orders only)

· US power cord (North American orders only)

· US English documentation

· Integrated Advanced Server Management features, including ServerWORKS Manager kit.

· Factory installed software

· Tru64 UNIX Unlimited User license, Server Extension license, Internet AlphaServer System software kit, or 

· OpenVMS base license with System Manager license, Enterprise Integrated Package

· Hardware Warranty: Three-year on-site hardware with next business day response

· Software Warranty: 90 day SPD conformance with advisory telephone support for TRU64 UNIX and OpenVMS

· Rackmount/Desktop kit included

Step 1a(Tru64 UNIX Systems

· Systems include pre-installed Tru64 UNIX, Base license, Unlimited User license, Server Extension license and Internet Access Server Software (IASS) media and license.

· Systems include factory installed software (FIS) Tru64 UNIX V4.0F on hard disk drive.

· Systems ordered in North America include a 104/105 key PC style US English keyboard and 120 V power cord.

· Selection of a country-specific keyboard and power cord is mandatory for all non North American orders.

· 
· Tru64 UNIX media and documentation must be ordered separately.

Order Number
Memory
Hard Disk Drive
Ethernet
Graphics
Monitor

DA-PB10A-AB
128 MB
Mandatory
Onboard
Optional
Optional

DA-PB10A-AC
256 MB
Mandatory
Onboard
Optional
Optional

DA-PB10A-AD
512 MB
Mandatory
Onboard
Optional
Optional

Step 1b(OpenVMS Systems

· Systems include pre-installed OpenVMS, Base license, and Enterprise Integration Package.

· Systems include Factory Installed Software (FIS) OpenVMS V7.1-2 on the hard disk.

· OpenVMS variants ordered in North America include a 108-key, VT style US English keyboard and 120 V power cord.

· Selection of a keyboard and power cord is mandatory for all non North American orders.

· OpenVMS media and documentation must be ordered separately.

Order Number
Memory
Hard Disk Drive
Ethernet
Graphics
Monitor

DY-PB10A-AB
128 MB
Mandatory
Onboard
Optional
Optional

DY-PB10A-AC
256 MB
Mandatory
Onboard
Optional
Optional

DY-PB10A-AD
512 MB
Mandatory
Onboard
Optional
Optional

Step 1c(Mandatory Options

· Selection of a SCSI System Disk drive is mandatory.  Systems support  a total of three internal disk drives.

· System supports 3.5-inch x 1-inch internal drives only.

· SCSI cable kit is required for internal SCSI disks and tape.

SCSI Disks

Selection of a SCSI adapter and a SCSI cable kit are mandatory for SCSI disk drives.

Order Number
System Disk 

RZ1CF-WA
4.3 GB 7200 rpm Ultra SCSI hard disk drive

DS-RZ2DA-WA
9.1 GB 7200 rpm Ultra SCSI hard disk drive

DS-RZ1DD-WA
9.1 GB 10,000 rpm Ultra SCSI hard disk drive

SCSI Adapters

Order Number
SCSI Adapter 

KZPBA-CA
PCI-based one port UltraSCSI Single-ended adapter

KZPCM-DA
PCI-based two port UltraSCSI Single-ended adpater and 10/100 Mbit Ethernet Combo adapter

SCSI Cable Kit

Order Number
SCSI Cable Kit 

3X-BN50A-01
SCSI cable kit (includes SCSI cable and SCSI LED cable)

Step 2(Memory

· Systems include one memory kit.

· Systems support a maximum of two memory kits.  Each kit includes two industry standard DIMMs with ECC support.

· Systems support maximum of 1 GB memory.

Order Number


3X-MS310-AA
64 MB, Two 200 pin, 100 MHz, 32 MB DIMMs

3X-MS310-BA
128 MB, Two 200 pin, 100 MHz, 64 MB DIMMs

3X-MS310-CA
256 MB, Two 200 pin, 100 MHz, 128 MB DIMMs

3X-MS310-DA
512 MB, Two 200 pin, 100 MHz, 256 MB DIMMs

Step 3(Storage 

Step 3a(Internal Storage

· Systems include one 600 MB 32X CD-ROM drive and one 1.44 MB diskette drive in dedicated drive slot.

· Additional internal storage supported: 

· Three 1” hard disk drive bays (including mandatory selection from step 1c)

· One removable 5.25-inch half-height removable media bay.  Note: one 3.5-inch x 1-inch hard disk drive is supported in removable media bay.

· SCSI cable kit is required for SCSI disks or tape

SCSI disk drives

Order Number
System Disk 

RZ1CF-WA
4.3 GB 7200 rpm Ultra SCSI hard disk drive

DS-RZ2DA-WA
9.1 GB 7200 rpm Ultra SCSI hard disk drive

DS-RZ1DD-WA
9.1 GB 10,000 rpm Ultra SCSI hard disk drive

SCSI Cable Kit

3X-BN50A-01
SCSI cable kit (includes SCSI cable and SCSI LED cable)

Removable Media Devices

TLZ10-LB
TLZ10 DAT 4 mm Internal Tape Drive (requires SN-PBXKP-BA wide to narrow SCSI adapter)

SN-PBXKP-BA
Wide to Narrow SCSI adapter

Step 3b(External Storage

Tabletop DAT Tape Drives

TLZ9L-DB
32/64 GB Tabletop DAT Loader

TKZ9F-TA
5,7,10,14GB 8 mm tbitp, SE SCSI

TLZ9L-VA
32/64 GB DAT loader in Carrier

Step 3b(External Storage (continued)
Controllers/Adapters


Maximum # Supported


Order Number

Description
Tru64 UNIX 
V4.0E
OpenVMS
V7.1-2

KZPBA-CA
PCI-based one port UltraSCSI Single-ended adapter, uses 1 PCI slot.  Internal 68-pin and 50-pin HD connectors and external 68-pin HD connector.
2
2

KZPBA-CB
PCI-based one port UltraSCSI Differential adapter, uses 1 PCI slot, external 68-pin HD connector.
2
2

BN38C-xx
68-pin HD male to VHDCI male UltraSCSI cable; required for each external UltraSCSI shelf attached to KZPBA-CA/CB adapter.



Step 4(Graphics Options 

SN-PBXGB-AA
PowerStorm 3D30 2D/3D 8-plane adapter

Step 5(Monitors 

Graphics monitors other than those listed below can be used if compatible with SVGA graphics ordered with a system.

Note:
Selection of a video extension cable and a country specific power cord is mandatory for all monitor variants.
See Step 5a for video extension cable.
See Step 5b for country specific power cords.

3R-VRQV5-24/11 
15" Corporate Series auto-scan color monitor, VGA to 1024 x 768 @ 85 Hz. 
Requires mandatory selection of country specific power cord for all variants.
–24 = Northern Hemisphere without power cord
–11 = Southern Hemisphere without power cord

SN-VRQP7-24/23
17” (16.0” viewable image size) professional series auto-scanning color monitor, Trinitron CRT, 0.25 mm aperture grill pitch, VGA to 1280 x 1024 at 75 Hz, TCO 95, MPR-II, Energy Star, attached 1.8 meter video cable.  Requires mandatory selection of video extension cable and country specific power cord for all variants.
–24 = Northern Hemisphere without power cord
–23 = Southern Hemisphere without power cord

SN-VRQP1-24/23
21" (19.6" viewable image size) auto-scanning color monitor, Trinitron CRT, 0.25 mm aperture grill pitch, VGA to 1600 x 1200 at 75 Hz NI, TCO 95, Energy Star, includes a 1.8 meter video cable.  Requires mandatory selection of video extension cable and country specific power cord for all variants.
–24 = Northern Hemisphere without power cord
–23 = Southern Hemisphere without power cord

Step 5a(Video Extension Cable

BN39C-02 
1.8 meter video extension cable.
Order one for each monitor ordered.

Step 5b(Monitor Power Cords

BN26J-1K
North America

3X-BN46F-02
Japan, 2.5 meters long, Dentori approved

BN19H-2E
Australia, New Zealand, 2.5 meters long

BN19C-2E
Central Europe, 2.5 meters long

BN19A-2E
UK, Ireland, 2.5 meters long

BN19E-2E
Switzerland, 2.5 meters long

BN19K-2E
Denmark, 2.5 meters long

BN19M-2E
Italy, 2.5 meters long

BN19S-2E
Egypt, India, South Africa, 2.5 meters long

BN18L-2E
Israel, 2.5 meters long

Step 6(Keyboards and Power Cords

Keyboards

· Systems ordered in North America include US English keyboard.

· Selection of a country specific keyboard is mandatory for all non North American orders.

Tru64 UNIX Keyboard
OpenVMS Keyboard
Country

SN-LKQ47-AA
LK461-A2
US/UK/English

SN-LKQ47-AB
LK461-AB
Belgian

SN-LKQ47-AC
LK461-AC
Canadian/French

SN-LKQ47-AD
LK461-AD
Danish

SN-LKQ47-AE

UK

SN-LKQ47-AF
LK461-AF
Finnish

SN-LKQ47-AG
LK461-AG
German

SN-LKQ47-AH
LK461-AH
Dutch

SN-LKQ47-AI
LK461-AI
Italian

SN-LKQ97-AJ

Japanese

SN-LKQ47-AK
LK461-AK
Swiss/French


LK461-AL
Swiss/German

SN-LKQ47-AM
LK461-AM
Swedish

SN-LKQ47-AN
LK461-AN
Norwegian

SN-LKQ47-AP
LK461-AP
French


LK461-AQ
Canadian/English

SN-LKQ47-AR

Latin-American

SN-LKQ47-AS
LK461-AS
Spanish

SN-LKQ47-AT
LK461-AT
Hebrew

SN-LKQ47-AV
LK461-AV
Portuguese

SN-LKQ47-AX

BHCSY

SN-LKQ47-BA

International

SN-LKQ47-BH
LK461-BH
Greek

SN-LKQ47-BI

Taiwanese

SN-LKQ47-BK

Korean


LK461-BL
Romanian

SN-LKQ47-BP
LK461-BP
Polish

Step 6(Keyboards and Power Cords (continued)

SN-LKQ47-BQ
LK461-BQ
Hungarian

SN-LKQ47-BR

Arabic

SN-LKQ47-BT
LK461-BT
Cyrillic

SN-LKQ47-BU
LK461-BU
Turkish

SN-LKQ47-BV
LK461-BV
Czech


LK461-BW
Turkish-French


LK461-BY
Yugoslavian

SN-LKQ47-CB

Thai

SN-LKQ47-CV

Simplified Chinese

SN-LKQ47-CZ
LK461-CZ
Slovak

Pedestal Power Cords

· Systems ordered in North America include one BN26J-1K 120 V North American power cord.  

· Selection of a country specific power cord is mandatory for all non North American orders.

3X-BN46K-2E
North America, 2.5 meters long

3X-BN46G-02
Japan, 2.5 meters long, Dentori approved

BN19H-2E
Australia, New Zealand, 2.5 meters long

BN19C-2E
Central Europe, 2.5 meters long

BN19A-2E
UK, Ireland, 2.5 meters long

BN19E-2E
Switzerland, 2.5 meters long

BN19K-2E
Denmark, 2.5 meters long

BN19M-2E
Italy, 2.5 meters long

BN19S-2E
Egypt, India, South Africa, 2.5 meters long

BN18L-2E
Israel, 2.5 meters long

Rackmount Power Cords

BN20Z-4E
4.5m rackmount power cord for North America

3X-BN46L-4E
4.5 m rackmount power cord for non-NA

3X-BN46D-4E
4.5m rackmount power jumper cord, Japan, Dentori

Step 7—Cabinet Enclosures and Accessories

· DS10 systems ship with a rackmount kit included.

M-series Top Gun Blue Cabinet Enclosures are available in three sizes:

· 43” high Cabinet Enclosure (H9A11-xx) 20U or 35.0” of usable vertical space.

· 67” high Cabinet Enclosure (H9A10-xx) 31U or 54.2” of usable vertical space.

· 79” high Cabinet Enclosure (H9A15-xx) 38U or 66.5” of usable vertical space.

Cabinet Configuration Chart

Cabinet Enclosure
Mounting Kit
Cabinet Height and Space
System
Shelves
Ballast Kit

H9A11-MC/MD
Included in Rackmount System
35.0 inches
20U
1
0
1





2
6
1





3
4
0





4
2
0





5
2
0





6
0
0

Step 7—Cabinet Enclosures and Accessories (continued)

Cabinet Enclosure
Mounting Kit
Height
Space
System
Shelves
Ballast Kit

H9A10-MC/MD
Included in Rackmount System
54.2 inches
31U
1
0
2





2
0
2





3
6
1





4
6
1





5
6
1





6
6
0





7
4
0





8
2
0





9
2
0





10
0
0

Cabinet Enclosure
Mounting Kit
Height
Space
System
Shelves
Ballast Kit

H9A15-MC/MD
Included in Rackmount System
66.5 inches
38U
1
N/A
N/A





2
0
2





3
6
2





4
6
1





5
6
1





6
6
1





7
6
0





8
6
0





9
4
0





10
4
0





11
2
0





12
0
0

M-series Cabinet Enclosure Options

M-series Cabinet Enclosures require the mandatory selection of:

· H9A11, H9A10, or H9A15 Cabinet 120V or 240V variant

· Cabinet front door based on cabinet height (optional on H9A11 cabinet)

· Factory Integration

· Ballast kits as required

Cabinet Enclosures

H9A11-MC
120 V one 24 A controller with 10 NEMA 5-15R outlets, input power plug NEMA L5-30P, Americas, Asia Pacific.
Includes rear door, 1 H7600-AA power distribution unit, trim kit, and front filler bezel kit

H9A11-MD
240 V one 16 A controller with 12 IEC C13 outlets, input power plug IEC 309, 16A, Europe
Includes rear door, 1 H7600-DB power distribution unit, trim kit, and front filler bezel kit

H9A10-MC
120 V dual 24 A controller, each with 10 NEMA 5-15R outlets, input power plug NEMA L5-30P, Americas, Asia Pacific.
Includes rear door, top fan tray, and 2 H7600-AA power distribution units

H9A10-MD
240 V dual 16 A controller, each with 12 IEC C13 outlets, input power plug IEC 309, 16A, Europe.
Includes rear door, top fan tray, and 2 H7600-DB power distribution units

Step 7—Cabinet Enclosures and Accessories (continued)

H9A15-MC
120 V dual 24 A controller, each with 10 NEMA 5-15R outlets, input power plug NEMA L5-30P, Americas, Asia Pacific.
Includes rear door, top fan tray, and 2 H7600-AA power distribution units

H9A15-MD
240 V dual 16 A controller, each with 12 IEC C13 outlets, input power plug IEC 309, 16A, Europe.
Includes rear door, top fan tray, and 2 H7600-DB power distribution units

For custom integration of systems, options, and additional Compaq and third party devices, or for other cabinets, doors, and trim kits of a different color, contact Compaq CustomSystems at 1-800-AT-COMPAQ (1-800-282-6672), or contact your local authorized Compaq Certified Integration Partner

Cabinet Front Door

H9C10-TF
Top Gun Blue front door for H9A10-MC/MD Cabinet Enclosure

H9C15-TF
Top Gun Blue front door for H9A15-MC/MD Cabinet Enclosure

H9C11-EB1
Top Gun Blue front door for H9A11-MC/MD Cabinet Enclosure

180 Selection of front door for H9A11 cabinet is optional, cabinet includes front trim kit and filler bezel kit.

Shelves supported in M-series Cabinets with front doors (H9A10/M9A15) 

BA36R-RC
StorageWorks UltraSCSI single bus, single-ended shelf with 180 W power supply and RETMA mounting kit

BA36R-RD
StorageWorks UltraSCSI split bus, single-ended shelf with 180 W power supply and RETMA mounting kit

BA35R-MR
StorageWorks controller shelf with RETMA mounting kit and NEMA power for H9A10-MC, 
H9A15-MC cabinets (Americas, Asia Pacific)
Order BN30B-02 power cord for H9A10-MD, H9A15-MD cabinets (Europe)

CK-BA35X-HH
180 W power supply

Shelves supported in M-series Cabinets with trim kit (H9A11) 

BA35C-AA
StorageWorks UltraSCSI single bus, single-ended shelf with 180 W power supply and RETMA mounting kit—blue bezel

BA35C-BA
StorageWorks UltraSCSI split bus, single-ended shelf with 180 W power supply and RETMA mounting kit—blue bezel

Factory Integration

YS-ASCAA-AA
Factory Integration of systems and storage devices assembled and configured in cabinet enclosure for H9A10/15 cabinet enclosures.

YS-ASCAA-AB
Factory Integration of systems and storage devices assembled and configured in cabinet enclosure for H9A11 cabinet enclosures.

Ballast Kit

Ballast is dead weight that is added to a cabinet at specific locations for specific configurations to meet safety regulations for installation, operation, and service.  See Cabinet Configuration Chart for ballast kit requirements.

H9C10-MC
Two 35-40 lbs. steel plates

For additional information on M-series Cabinet Enclosures visit www.digital.com/alphaserver/products/m-series.html 

Compaq CustomSystems Support

Cabinets containing systems and storage must be assembled by qualified assemblers: Compaq Manufacturing, Compaq’s Customs Systems Group, or an authorized Compaq Certified Integration Partner.

For custom integration of systems, options, and additional Compaq and third party devices, or for other cabinets, doors, and trim kits of a different color, contact Compaq CustomSystems at 1-800-AT-COMPAQ (1-800-282-6672), or contact your local authorized Compaq Certified Integration Partner.

Step 8(Network Options 

· Systems include dual embedded 10/100 BaseT ethernet ports (BN25G-xx cable must be ordered separately).

· Additional ethernet ports can be added using available PCI slots


Maximum # Supported


Order Number

Description
Tru64 UNIX V4.0E
OpenVMS
V7.1-2

DE500-BA
PCI-based 10/100-Mbit 1 port Fast Ethernet controller (Twisted Pair)
2
2

DE504-BA
PCI-based 10/100-Mbit 4 Port Fast Ethernet controller (Twisted Pair)
2
2

BN25G-xx
Cable for 10BaseT Twisted Pair connection



DE500-FA
PCI-based 10/100-Mbit Fast Ethernet controller (MMF)_
2
2

BN34B-xx
SC-SC Dual FO cable, mm, pp. xx = length in meters



Step 9(Software 

Step 9a(Tru64 UNIX Software 

· Tru64 UNIX systems include Tru64 UNIX base license, Server Extension, Internet AlphaServer Software kit and Unlimited User License.

· Tru64 UNIX systems require operating system media and documentation for first system on site.

Tru64 UNIX Media and Documentation(required for first system on site

QA-MT4AA-H8
Tru64 UNIX Media & Documentation kit

QA-MT4AA-GZ
Tru64 UNIX Full Documentation kit

TruCluster Available Server License

QL-05SAE-AA
TruCluster Available Server License

Tru64 UNIX Layered Products CD-ROM

QA-054AA-H8
Layered products media and documentation for Tru64 UNIX on CD-ROM

DECnet for Tru64 UNIX 

QL-MTJAE-AA
DECnet/OSI end-system license for Tru64 UNIX

QL-MTKAE-AA
DECnet/OSI extended function license for Tru64 UNIX

Open3D for Tru64 UNIX CD-ROM

QA-0AFAA-H8
Open3D for Tru64 UNIX on CD-ROM, includes graphics drivers

OpenVMS Packaged and Base systems require operating system media and documentation for first system on site.

· Software Processor Code = E

Step 9b(OpenVMS Software

OpenVMS Media and Documentation

QA-MT1AD-H8
OpenVMS V7.1-1H1 Media on CD-ROM

QA-MT1AA-H8
OpenVMS V7.1 media and on-line documentation on CD-ROM

QA-001AA-GZ
OpenVMS hardcopy documentation

Step 9b(OpenVMS Software  (continued)

OpenVMS Layered Products CD-ROM

QA-03XAA-H8
Layered products media and documentation for OpenVMS on CD-ROM

Open3D for OpenVMS CD-ROM

QA-0ADAA-H8
Open3D for OpenVMS on CD-ROM, includes graphics drivers

OpenVMS Concurrent Use Licenses

· OpenVMS Concurrent Use Licenses are not specific to a single system and can be moved between systems at user discretion. OpenVMS Concurrent Use Licenses can also be shared in a mixed OpenVMS VAX and OpenVMS Alpha Cluster.  The OpenVMS traditional unlimited use license is system specific and can only be used on one single system at a time. It can not be shared between systems or in an OpenVMS VAX or OpenVMS Alpha Cluster.

QL-MT3AA-3B
OpenVMS Concurrent Use 1-user license

QL-MT3AA-3C
OpenVMS Concurrent Use 2-user license

QL-MT3AA-3D
OpenVMS Concurrent Use 4-user license

QL-MT3AA-3E
OpenVMS Concurrent Use 8-user license

QL-MT3AA-3F
OpenVMS Concurrent Use 16-user license

QL-MT3AA-3G
OpenVMS Concurrent Use 32-user license

QL-MT3AA-3H
OpenVMS Concurrent Use 64-user license

QL-MT3AA-3J
OpenVMS Concurrent Use 128-user license

QL-MT3AA-3K
OpenVMS Concurrent Use 256-user license

QL-MT2AE-AA
OpenVMS traditional unlimited user license

Step 10(Hardware and Software Supplemental Services

Hardware Supplemental Services

FM-MK4HR-36
3 YR 5X9/4HR, AS1000/AlphaServer DS10

FM-MK4HR-60
5 YR 5X9/HR, AS1000 /AlphaServer DS10

FM-MK512-36
3 YR 5X12/4HR, AS1000 /AlphaServer DS10

FM-MK512-60
5 YR 5X12/4HR, AS1000 /AlphaServer DS10

FM-MK616-36
3 YR 6X16/4HR, AS1000/AlphaServer DS10 

FM-MK616-60
5 YR 6X16/4HR, AS1000/AlphaServer DS10 

FM-MK724-36
3 YR 7X24/4HR, AS1000/AlphaServer DS10 

FM-MK724-60
5 YR 7X24/4HR, AS1000/AlphaServer DS10 

FM-MKXHW-60
YR 4-5 5X9/N.D., AS1000/AlphaServer DS10 

Step 10(Hardware and Software Supplemental Services(continued)

Software Supplemental Services

FM-D10V9-12
1 yr, DS10 OpenVMS Brz9x5

FM-D10V9-36
3 yr, DS10 OpenVMS Brz9x5

FM-D10V9-60
5 yr, DS10 OpenVMS Brz9x5

FM-D10VS-12
1 yr, DS10 OpenVMS Brz24x7

FM-D10VS-36
3 yr, DS10 OpenVMS Brz24x7

FM-D10VS-60
5 yr, DS10 OpenVMS Brz24x7

FM-D10VN-12
1 yr, DS10 OpenVMS Node

FM-D10VN-36
3 yr, DS10 OpenVMS Node

FM-D10VN-60
5 yr, DS10 OpenVMS Node

FM-D10U9-12
1 yr, DS10 Tru64 UNIX Brz9x5

FM-D10U9-36
3 yr, DS10 Tru64 UNIX Brz9x5

FM-D10U9-60
5 yr, DS10 Tru64 UNIX Brz9x5

FM-D10US-12
1 yr, DS10 Tru64 UNIX Brz24x7

FM-D10US-36
3 yr, DS10 Tru64 UNIX Brz24x7

FM-D10US-60
5 yr, DS10 Tru64 UNIX Brz24x7

FM-D10UN-12
1 yr, DS10 Tru64 UNIX Node

FM-D10UN-36
3 yr, DS10 Tru64 UNIX Node

FM-D10UN-60
5 yr, DS10 Tru64 UNIX Node

Systems Specifications 

Height
5 in (13 cm)

Width
17 in (43 cm)

Depth
18 in (46 cm)

Weight (fully loaded)
36 lb (16.3 kg)

[image: image3.jpg]SN .
7\
Pre h“\! L ‘5
‘l%’:!!'; B
& ‘45'/
{ //'
PK0332A-99

Z.

0 /é;\.m.m ‘
iz
9/

REAR

PCI riser card

CPU and fan

Power supply

Space for optional disk
CD-ROM

Optional bay

System disk

Control panel

Floppy diskette drive
Concealed optional bay

1
2
3
4
5
6
7
8
9
10
11




[image: image4.jpg][/0O PORTS

N

g’mﬁ e # i*
@ =

! e W W

J
1. COM1 and RMC port 5. USB ports
2. COM2 port 6. Parallel port
PK0334-99 3. Mouse 7. Ethernet A
4. Keyboard 8. Ethernet B





�





�





�








1.14
 AlphaServer

AlphaServer 1.15

_977841905.wmf

