Compaq AlphaStation XP1000/667

Compaq AlphaStation XP1000/667



[image: image1.png]



Compaq AlphaStation XP1000/667 
V1.1(16 August 1999
Systems and Options Catalog
Product Description
Compaq AlphaStation XP1000/667 is available from qualified, Compaq Workstation Sales Specialists and through distribution partners and can be ordered as either a 2-5-2 or as a  6-3 part number.  A list of eligible partners is available through  the local Compaq Sales Office.

Compaq AlphaStations XP1000/667 are available as:

· System Building Blocks(SBB)/Configure to order(CTO) models—designed for “a la carte” ordering and distributor integration.  System Building Blocks contain a 667 MHz system kernel, 4 MB 222 MHz external cache memory, and either, operating system- less (LINUX ready), or factory installed Windows NT or Tru64 UNIX operating system.  System Building Blocks require a minimum of 128 MB memory, one hard disk drive and a country kit.  Selection of a graphics adapter is mandatory for Windows NT SBB.  All system building blocks require the selection of a country specific keyboard for full system operation. 
· Packaged Systems/Build to order (BTO) models—designed as complete systems for shipment to end-users.  Packaged Systems contain a 667 MHz system kernel, 4 MB 222 MHz external cache, a minimum of 128 MB memory, a minimum 9.1 SCSI disk drive, single channel wide ultra-2 SCSI (LVD) adapter, graphics adapter, country kit (for all but European packages) and factory installed Windows NT or Tru64 UNIX operating system. All packaged systems include a country kit and country specific keyboard except European packages. Selection of a country kit is mandatory for European systems. Additional options will ship separate and will not be factory integrated into packaged systems.

Compaq AlphaStation XP1000/667 supports five I/O expansion slots on two buses: two 64-bit PCI, two 32-bit PCI, and one PCI/ISA 32-bit combination slot.  System provides six internal storage device bays, including a floppy diskette, a CD-ROM, two additional 5.25-inch removable media devices, and two hard disk drive bays.  Onboard options include an UltraSCSI adapter, a 16-bit Soundblaster TM audio compatible, and 10/100 Ethernet (Twisted pair) network interface.  A maximum of 2 GB of memory is supported in the system enclosure.

Compaq AlphaStation XP1000/667 comes standard with a three-year limited hardware warranty.  Year one features on-site service with second day response; years two and three are return to Compaq service, parts only.  This warranty represents one of the most comprehensive in the industry.

Compaq and the names of Compaq products referenced herein are either trademarks and/or service marks or registered trademarks and/or service marks of Compaq.

DIGITAL is a Trademark of Compaq Computer Corporation. 

Microsoft, Windows, Windows NT, SQL Server, Office and BackOffice are either trademarks or registered trademarks of Microsoft Corporation.

Intel and Pentium are registered trademarks of Intel Corporation.

Other product and company names mentioned herein may be trademarks and/or service marks of their respective owners.

Step 1—Compaq AlphaStation XP1000/667

Compaq AlphaStation XP1000/667 includes

Tower enclosure with

· Alpha 21264A (EV67) 667 MHz CPU with
 64 KB data cache, 64 KB instruction cache and
 4 MB (222 MHz)  external cache 

· Five Expansion slots on two buses

Two 64-bit PCI

Two 32-bit PCI

· One PCI/ISA 32-bit combination slot

· Two memory option banks

Four DIMM slots per bank

Maximum of 2 GB 

· Six Storage bays

One diskette drive bay

One CD-ROM drive bay

One 3.5-inch x 1.6-inch and one 3.5-inch x 1.0-inch removable media bays

Two 3.5-inch x 1.0-inch hard disk drive bays

· 400 W power supply

· Onboard UltraSCSI adapter. PCI-based single-channel wide Ultra-2 SCSI (LVD) adapter optional (included in all packaged systems)
· Onboard 16-bit SoundblasterTM compatible audio

· Onboard 10/100 Ethernet (Twisted pair) (ThinWire and Thick wire optional)

· Integrated I/O comprising parallel port, two serial ports, two USB ports(available from NT only) , audio in, audio out, headphone, microphone, mouse and keyboard ports.

· 1.44 MB diskette drive

· 600 MB 32X CD-ROM drive

· LINUX-ready (no operating system installed ) or   

· Factory Installed Software : either
· Windows NT 4.0 includes: Windows NT base license, with Service Pack 3 (or higher) or
· Tru64 UNIX V4.0F (minimum) includes: Tru64 UNIX base license, two-user UNIX license, and COMPAQ’S Open3D license. 

Step 1a—System Building Blocks – CTO Models

· Manadatory components required for SBBs:

· Minimum of 128 MB memory

· One hard disk drive

· Country Kit

· Selection of a graphics adapter is mandatory for Windows NT SBBs.

· Selection of a graphics adapter is optional for LINUX-ready and Tru64 UNIX SBBs.

· All system building blocks require a country specific keyboard.  Selection of a keyboard is not mandatory but is essential for full system operation.

· Additional components in Steps 3 through 11 are optional and will be factory integrated when ordered with a system building block.

667 MHz CTO/System Building Blocks—No OS (LINUX-ready)

Order Number
Memory / Cache
Graphics
Hard Disk Drive
Country Kit 

SN-E2G6W-L2
143863-B21
Mandatory/4MB
Optional
Mandatory
Mandatory

667 MHz CTO/System Building Blocks—CIP only

Order Number
Memory / Cache
Graphics
Hard Disk Drive
Country Kit 

SN-E2G6W-CA
129118-B21
Mandatory/4MB
Optional
Mandatory
Mandatory

667 MHz CTO/System Building Blocks—Windows NT 4.0

Order Number
Memory / Cache
Graphics
Hard Disk Drive
Country Kit 

SN-E2G6W-V2
143863-B21
Mandatory / 4 MB
Mandatory
Mandatory
Mandatory

667 MHz CTO/System Building Blocks—Tru64 UNIX

Order Number
Memory / Cache
Graphics
Hard Disk Drive
Country Kit 

SN-E2G6W-32
143862-B21
Mandatory / 4 MB
Optional
Mandatory
Mandatory

Step 1b—Packaged Systems – BTO models

· All packaged systems include a country kit and country specific keyboard except European packages

· Selection of country Kit from step 2 is mandatory for European packages.

· Single-channel wide Ultra-2 SCSI (LVD) adapter (SN-KZPCA-CB) installed (one PCI-slot) in all packaged systems.
· Additional options will not be factory integrated into packaged systems and will ship as spares.

667 MHz BTO/Packaged Systems—Windows NT 4.0 
Order Number
Memory / Cache
Graphics
10K RPM Hard Disk 
US Country Kit 

SN-E2G6Q-VH
127346-001
256 MB / 4 MB
ELSA GLoria Synergy 
9.1 GB SCSI disk
Included

SN-E2G6Q-VI
127346-002
256 MB / 4 MB
PowerStorm 300
9.1 GB SCSI disk 
Included

SN-E2G6Q-VJ
127346-003
256 MB / 4 MB
PowerStorm 350
9.1 GB SCSI disk
Included

Order Number
Memory / Cache
Graphics
10K RPM Hard Disk 
European Country Kit 

SN-E2G6Q-EH
127346-021
256 MB / 4 MB
ELSA GLoria Synergy
9.1 GB SCSI disk 
Mandatory

SN-E2G6Q-EI
127346-022
256 MB / 4 MB
PowerStorm 300
9.1 GB SCSI disk 
Mandatory

SN-E2G6Q-EJ
127346-023
256 MB / 4 MB
PowerStorm 350
9.1 GB SCSI disk 
Mandatory

Order Number
Memory / Cache
Graphics
10K RPM Hard Disk 
Japan Country Kit 

SN-E2G6Q-JH
127346-291
256 MB / 4 MB
ELSA GLoria Synergy 
9.1 GB SCSI disk 
Included

SN-E2G6Q-JI
127346-292
256 MB / 4 MB
PowerStorm 300
9.1 GB SCSI disk 
Included

SN-E2G6Q-JJ
127346-293
256 MB / 4 MB
PowerStorm 350
9.1 GB SCSI disk 
Included

Order Number
Memory / Cache
Graphics
10K RPM Hard Disk 
Japan/English Country Kit 

SN-E2G6Q-HH
127346-391
256 MB / 4 MB
ELSA GLoria Synergy 
9.1 GB SCSI disk 
Included

SN-E2G6Q-HI
127346-392
256 MB / 4 MB
PowerStorm 300
9.1 GB SCSI disk 
Included

SN-E2G6Q-HJ
127346-393
256 MB / 4 MB
PowerStorm 350
9.1 GB SCSI disk 
Included

Order Number
Memory / Cache
Graphics
10K RPM Hard Disk 
Chinese Country Kit 

SN-E2G6Q-CH
127346-371
256 MB / 4 MB
ELSA GLoria Synergy
9.1 GB SCSI disk 
Included

SN-E2G6Q-CI
127346-372
256 MB / 4 MB
PowerStorm 300
9.1 GB SCSI disk 
Included

SN-E2G6Q-CJ
127346-373
256 MB / 4 MB
PowerStorm 350
9.1 GB SCSI Disk 
Included

667 MHz BTO/Packaged Systems—Tru64 UNIX

Order Number
Memory / Cache
Graphics
10K RPM Hard Disk 
US Country Kit 

SN-E2G6Q-AA
127346-004
256 MB / 4 MB
ELSA GLoria Synergy 
9.1 GB SCSI disk 
Included

SN-E2G6Q-AB
127346-005
256 MB / 4 MB
PowerStorm 300
9.1 GB SCSI disk 
Included

SN-E2G6Q-AC
127346-006
256 MB / 4 MB
PowerStorm 350
9.1 GB SCSI Disk 
Included

Order Number
Memory / Cache
Graphics
10K RPM Hard Disk 
European Country Kit 

SN-E2G6Q-EA
127346-024
256 MB / 4 MB
ELSA GLoria Synergy 
9.1GB SCSI disk 
Mandatory

SN-E2G6Q-EB
127346-025
256 MB / 4 MB
PowerStorm 300
9.1 GB SCSI disk 
Mandatory

SN-E2G6Q-EC
127346-026
256 MB / 4 MB
PowerStorm 350
9.1 GB SCSI Disk 
Mandatory

Order Number
Memory / Cache
Graphics
10K RPM Hard Disk 
Japan Country Kit 

SN-E2G6Q-JA
127346-294
256 MB / 4 MB
ELSA GLoria Synergy 
9.1 GB SCSI disk 
Included

SN-E2G6Q-JB
127346-295
256 MB / 4 MB
PowerStorm 300
9.1 GB SCSI disk 
Included

SN-E2G6Q-JC
127346-296
256 MB / 4 MB
PowerStorm 350
9.1 GB SCSI Disk 
Included

Step 1b—Packaged Systems – BTO models (continued)

667 MHz BTO/Packaged Systems—Tru64 UNIX(continued)

Order Number
Memory / Cache
Graphics
10K RPM Hard Disk 
Japan/English Country Kit 

SN-E2G6Q-HA
127346-394
256 MB / 4 MB
ELSA GLoria Synergy 
9.1 GB SCSI disk 
Included

SN-E2G6Q-HB
127346-395
256 MB / 4 MB
PowerStorm 300
9.1 GB SCSI disk 
Included

SN-E2G6Q-HC
127346-396
256 MB / 4 MB
PowerStorm 350
9.1 GB SCSI disk 
Included

Order Number
Memory / Cache
Graphics
10K RPM Hard Disk 
Chinese Country Kit 

SN-E2G6Q-CA
127346-374 
256 MB / 4 MB
ELSA GLoria Synergy 
9.1 GB SCSI disk 
Included

SN-E2G6Q-CB
127346-375
256 MB / 4 MB
PowerStorm 300
9.1 GB SCSI disk 
Included

SN-E2G6Q-CC
127346-376
256 MB / 4 MB
PowerStorm 350
9.1 GB SCSI Disk 
Included

667 MHz BTO/Packaged Systems –No OS (LINUX-ready)

Order Number
Memory / Cache
Graphics
10K RPM Hard Disk 
 Country Kit 

SN-E2G6Q-VL
127346-007
256 MB / 4 MB
none 
9.1 GB SCSI disk 
US included

SN-E2G6Q-EL
127346-027 
256 MB / 4 MB
None
9.1 GB SCSI disk 
European mandatory

SN-E2G6Q-JL
127346-297
256 MB / 4 MB
None
9.1 GB SCSI Disk 
Japanese included

SN-E2G6Q-HL
127346-397
256 MB / 4 MB
None
9.1 GB SCSI disk 
Japanese English included

SN-E2G6Q-CL
127346-377
256 MB / 4 MB
None
9.1 GB SCSI disk 
Chinese included

Step 2—Country Kit

· LINUX-ready or Tru64 UNIX universal country kits include two country-specific power cords, user documentation, and mouse.  Tru64 UNIX license is included with system building blocks and packaged systems.

· Windows NT country kits include two country-specific power cords, user documentation, mouse, operating system license, media and documentation.

· The following Factory Installed Software (FIS) languages are the only languages delivered.  

· English, Japanese, French, Italian, German, Spanish and Dutch.
· Additional Windows NT languages may be purchased directly from Microsoft.

· Country kit is required for system to be operational.

· Only one country kit is allowed per system.

· Bulk system orders must include country kits equal to the total number of systems ordered.

Americas

United States, Canada, Taiwan, Philippines, Thailand, Japan
SN-PBB0P-AA   388123-001
English
SN-PBB0U-AA   388176-001
English

Canada
SN-PBB0P-AC   388123-121
French
SN-PBB0U-AC   388176-121
French

Europe

Denmark
SN-PBB0P-AD   388123-081
Danish
SN-PBB0U-AD   388176-081
English

Finland
SN-PBB0P-AF   388123-351
Finish
SN-PBB0U-AF   388176-351
English

Norway
SN-PBB0P-AN   388123-091
Norwegian
SN-PBB0U-AN   388176-091
English

Sweden
SN-PBB0P-AM  388123-101
Swedish
SN-PBB0U-AM   388176-101
English

Step 2—Country Kit (continued)

Country Supported
Windows NT 4.0
Country Kits
Windows NT 4.0 Media
Language
LINUX-ready or Tru64 UNIX 4.0F Country Kits
FIS and Documentation 

Language Supported

Europe (continued)

Germany
SN-PBB0P-AG   388123-041
German
SN-PBB0U-AG   388176-041
German

Netherlands
SN-PBB0P-AH   388123-331
Dutch
SN-PBB0U-AH   388176-331
Dutch

Italy
SN-PBB0P-AI    388123-061
Italian
SN-PBB0U-AI   388176-061
Italian

Switzerland/French
SN-PBB0P-AK   388123-301
French
SN-PBB0U-AK   388176-301
French

Switzerland/Germany
SN-PBB0P-AL   388123-311
German
SN-PBB0U-AL   388176-311
German

Switzerland/Italian
SN-PBB0P-AW  388123-321
Italian
SN-PBB0U-AW   388176-321
Italian

Switzerland/English
SN-PBB0P-CH   388123-111
English
SN-PBB0U-CH   388176-111
English

France, Belgium
SN-PBB0P-AP   388123-051
French
SN-PBB0U-AP   388176-051
French

Spain
SN-PBB0P-AS   388123-071
Spanish
SN-PBB0U-AS   388176-071
Spanish

United Kingdom
SN-PBB0P-AE   388123-031
English
SN-PBB0U-AE   388176-031
English

General European
SN-PBB0P-EC   388123-021
English
SN-PBB0U-EC   388176-021
English

Asia Pacific

Japan
SN-PBB0P-AJ   388123-291
Japanese
SN-PBB0U-AJ   388176-291
Japanese

Japan
SN-PBB0P-CC   388123-391
English
SN-PBB0U-CC   388176-391
English

Chinese/General Asia
SN-PBB0P-CA   388123-371
English
SN-PBB0U-CA   388176-371
English

Chinese
SN-PBB0P-CV  388123-AA1
Simplified Chinese
SN-PBB0U-CV   388176-AA1
Simplified Chinese

Australia/New Zealand
SN-PBB0P-CN   388123-011
English
SN-PBB0U-CN   388176-011
English

Step 3—Keyboards

· System building blocks and packaged systems do not include a country specific keyboard.  Selection of a keyboard is not mandatory but is essential for full system operation.

· Keyboards listed below are colored to match CPU and monitor. Keyboards purchased through distribution channels may not be the correct color match. 

Windows NT Keyboard
 LINUX-ready or Tru64 UNIX Keyboard
 Country

Americas

SN-LKQ97-AA     166515-006
 SN-LKQ47-AA     296435-006
 US/English

SN-LKQ97-AC     166515-126
 SN-LKQ47-AC     296435-126
 Canada/French

SN-LKQ97-AR     166515-166
 SN-LKQ47-AR     296435-166
 Latin-American/Spanish

SN-LKQ97-AU     —
—                            —
 Brazil

Europe

SN-LKQ97-AB     166515-186
 SN-LKQ47-AB     296435-186
 Belgian

SN-LKQ97-AD     166515-086
 SN-LKQ47-AD     296435-086
 Danish

SN-LKQ97-AE     166515-036
 SN-LKQ47-AE      296435-036
 UK

SN-LKQ97-AF     166515-356
 SN-LKQ47-AF     296435-356
 Finland

SN-LKQ97-AG     166515-046
 SN-LKQ47-AG     296435-046
 German

SN-LKQ97-AH     166515-336  
 SN-LKQ47-AH     296435-336
 Netherlands

SN-LKQ97-AI     166515-066
 SN-LKQ47-AI     296435-066
 Hebrew

SN-LKQ97-AK     166515-116
 SN-LKQ47-AK     296435-116
 Italy

SN-LKQ97-AM     166515-106
 SN-LKQ47-AM     296435-106
 Switzerland

SN-LKQ97-AN     166515-096
 SN-LKQ47-AN     296435-096
 Sweden

SN-LKQ97-AP     166515-056
 SN-LKQ47-AP     296435-056
 Norway

SN-LKQ97-AS     166515-076
 SN-LKQ47-AS     296435-076
 France

Step 3—Keyboards (continued)

Windows NT Keyboard
 LINUX-ready or Tru64 UNIX Keyboard
 Country

Europe (continued)

SN-LKQ97-AT     —
 SN-LKQ97-AT     —
 Spain



SN-LKQ97-AV      166515-136 
 SN-LKQ97-AV     296435-136
 Portugal

SN-LKQ97-AX     166515-B46
 SN-LKQ97-AX     296435-B46
 Bosnia, Herzagovnia, Croatia, Serbia,     Yugoslavia

SN-LKQ97-BH     166515-156
 SN-LKQ47-BH     296435-156
 Greece

SN-LKQ97-BP     166515-246
 SN-LKQ47-BP     296435-246
 Poland

SN-LKQ97-BQ     166515-216
 SN-LKQ47-BQ     296435-216
 Hungary

SN-LKQ97-BR     166515-176
 SN-LKQ47-BR     296435-176
 Arabic

SN-LKQ97-BT     166515-256
 SN-LKQ47-BT     296435-256
 Russia/Cyrillic

SN-LKQ97-BU     166515-146
 SN-LKQ47-BU     296435-146
 Turkey

SN-LKQ97-BV     166515-226
 SN-LKQ47-BV     296435-226
 Czech Republic

SN-LKQ97-CZ     166515-236
 SN-LKQ47-CZ     296435-236
 Slovak

Asia Pacific

SN-LKQ97-AJ     166515-296
—                           —
 Japan

SN-LKQ97-BI     166515-AB6
 SN-LKQ47-BI     296435-AB6
 Taiwan

SN-LKQ97-CV    —
 SN-LKQ47-CV    —
 China

SN-LKQ97-BK     166515-AD6
 SN-LKQ47-BK     296435-AD6
 South Korean

SN-LKQ97-CB     166515-286
 SN-LKQ47-CB     296435-286
 Thailand

SN-LKQ97-BA     166515-B36
 SN-LKQ47-BA     296435-B36
 International

Step 4—Memory 

· Selection of a minimum of 128 MB memory is mandatory for CTO/SBBs. 

· Systems support 128 MB to 2 GB system memory (two memory banks support four DIMMs per bank).

SN-MSP01-KA                388259-B21
128 MB ECC memory DIMMs (4 x 32MB DIMMs)

SN-MSP01-KB                388260-B21
256 MB ECC memory DIMMs (4 x 64MB DIMMs)

SN-MSP01-KC                388261-B21
512 MB ECC memory DIMMs (4 x 128MB DIMMs)

SN-MSP01-KD                388262-B21
1024 MB ECC memory DIMMs (4 x 256MB DIMMs)

Step 5(Storage

All systems include an onboard UltraSCSI adapter for internal drives only. Packaged Systems/BTO models also include a PCI-based Ultra-2 SCSI adapter (one SN-KZPCA-AA/136237-B21 factory installed).

Step 5a—Internal Storage

· System building blocks/ CTO models require one mandatory hard disk drive.

· Hard disk drive ordered with Windows NT or Tru64 UNIX SBB includes Factory installed software.

· Systems support up to four hard disk drives:

· Two 3.5-inch x 1.0-inch hard disk drive in dedicated hard disk drive bays.

· One 3.5-inch x 1.6-inch and one 3.5-inch x 1.0-inch hard disk drives supported in the 5.25-inch removable media bays.

Step 5a—Internal Storage (continued)

SCSI Disk Drives

SN-PBXRW-SC     146443-B21
9.1 GB Ultra2 LVD Wide  SCSI 7200 RPM hards disk drive 1.0 inch drive

SN-PBXRW-TB     146444-B21
18.2 GB Ultra2 LVD Wide SCSI 7200 RPM hard disk drive, 1.0-inch drive

SN-PBXRW-RB     388265-B21
4.3 GB Ultra2 LVD Wide SCSI 10K RPM hard disk drive, 1.0-inch drive

SN-PBXRW-VA     388266-B21
9.1 GB Ultra2 LVD Wide SCSI 10K RPM hard disk drive, 1.0-inch drive

SN-PBXRW-WA1    388268-B21
18.2 GB Ultra2 LVD Wide SCSI 10K RPM hard disk drive, 1.6-inch drive (includes fan kit)

1.
Maximum one per system, supported in the 5.25-inch removable media bays only.

Removeable Media

· Systems include one 5.25-inch 32X CD-ROM drive.

· Systems support two additional 5.25-inch removable media devices.  Note: 5.25-inch removable media bays also support hard disk drives.

· Limit of two ATAPI devices per system
· DVD is data support only (no MPEG-2 capability)
· DVD is backward compatible with CDROM media, (additional CDROM drive not needed)

SN-TLZ10-LK     388270-B21
12/24 GB 4 mm DAT internal SCSI tape drive, 1.6-inch front accessible bay

SN-TLZ09-LK     388269-B21
4/8 GB 4 mm DAT internal SCSI tape drive, 1.6-inch front accessible bay

SN-PBXRX-CA   146194-B21
100MB ATAPI ZIP Drive

SN-PBXRV-AA   146195-B21
6X/32X ATAPI DVD-ROM Drive (Data device only)

Step 5b—External Storage

An additional SCSI controller is required to support external drives or RAID arrays.

SCSI Controllers

· Systems include onboard UltraSCSI controller for internal disks only.

· Package systems include an additional PCI-based Wide Ultra-2 SCSI(LVD) adapter for improved internal disk performance.

· Select a maximum of  two PCI-based storage adapters or RAID controllers if additional external storage is required.

SN-KZPBA-CA     388272-B21
PCI-based single channel UltraSCSI Single-Ended adapter—requires one PCI slot.

SN-KZPBA-CB     388273-B21
PCI-based single channel UltraSCSI Differential adapter—requires one PCI slot.

SN-KZPAC-AA     388274-B21
PCI-based single channel RAID Array controller (UltraSCSI Single-ended) with 4 MB cache—requires one PCI slot.

SN-KZPAC-CA     388275-B21
PCI-based three channel RAID Array controller (UltraSCSI Single-ended) with 4 MB cache—requires one PCI slot.
Note: External use of the third channel requires one additional PCI bulkhead slot and a BN31K-0E SCSI cable/bulkhead assembly kit.

SN-KZPAC-CB     388276-B21
PCI-based three channel RAID Array controller (UltraSCSI Single-ended) with 8 MB cache—requires one PCI slot.
Note: External use of the third channel requires one additional PCI bulkhead slot and a BN31K-0E SCSI cable/bulkhead assembly kit.

SN-KZPCA-AA   136237-B21

PCI-based single channel Wide Ultra-2 SCSI (LVD) adapter 

Step 5b—External Storage (continued)

Expansion Boxes and SCSI cables

DS-BA356-KF                 108998-B21   
External StorageWorks deskside pedestal unit with 180 W power supply, dual speed blowers, UltraSCSI I/O personality module.  Configured for single bus.

BN38C-xx                        --
VHDCI male to 68-pin HD male SCSI cable.  Required to connect external StorageWorks UltraSCSI pedestal to KZPBA-xx adapters. (-01 one meter length, up to –020, 20 meter lengths).

BN37A-xx                       --
VHDCI male to VHDCI male SCSI cable. Required to connect external StorageWorks UltraSCSI pedestal to KZPAC-xx controller. (-01 one meter length, up to –025, 25 meter lengths).

Step 6—Graphics Options 

· Graphics adapter is mandatory for Windows NT system building blocks. 

· Graphics adapter is optional for Tru64 UNIX system building blocks. 

· System consists of two PCI buses with three PCI slots on Bus 0 and two PCI slots on Bus 1. 

· For optimal graphics performance all graphics adapters should be installed on Bus 0.  There will be a loss of performance on any adapter placed on PCI Bus 1.

· The first graphics adapter for single head systems must be installed on PCI Bus 0.

· Multi-head systems must be homogeneous, all ELSA GLoria Synergy or all PowerStorm 300, etc.

· Windows NT graphics options include Graphics Support Services Software license, media and documentation.

· Tru64 UNIX graphics options require DIGITAL Open3D media, included in factory installed software (FIS).

· Customers installing LINUX must refer to their supplier for graphic adapter support.



Maximum Graphics Adapters supported per system
Windows NT
Tru64 UNIX

SN-PBXGK-BB     400712-B21
ELSA GLoria Synergy graphics with 8 MB SGRAM
1/31
31

SN-PBXGI-AD     103455-B21
PowerStorm 4D51T 3D graphics with 16 MB texture memory
2
2

SN-PBXGD-AD     102454-B21
PowerStorm 300 Midrange 3D graphics with 15MB framebuffer memory and 16MB texture memory
2
1

SN-PBXGD-AE     102455-B21
PowerStorm 350 Midrange 3D graphics with 30 MB framebuffer memory and 32 MB texture memory
2
1

1.
ELSA GLoria Synergy: Windows NT supports one head for 2D/3D, two or three heads for 2D only.  Tru64 UNIX supports one to three heads for 2D only.

Step 7—Color Monitors 
Note: Country Kit includes two country-specific power cords, one for system and one for monitor.

Powerstorm 350 customers requiring to display the highest resolution stereo graphics (1280x1024 @ 120Hz) should          consider the Viewsonic P817 monitor as their current solution.

SN-VRQP7-23     307806-B23

SN-VRQP7-24     307806-B24
P75 COLOR MONITOR,  -23 =Southern hemisphere, -24 = Northern hemisphere
P75 Color Monitor -- 17" Trinitron monitor (16” viewable image size), 0.25mm aperture grille pitch, up to 1280 x 1024 resolution @ 75 Hz, OSD, USB pass through port, Plug & Play, TCO '95, Colorific Software, Asset Control, includes video cable.

SN-VRQP1-23     325600-B23

SN-VRQP1-24     325600-B24
P110 COLOR MONITOR, -23 = Southern Hemisphere, -24 = Northern hemisphere
P110 Color Monitor -- 21" Trinitron monitor (19.8” viewable image size), 0.25mm aperture grille pitch, up to 1600 x 1200 resolution @ 85 Hz, Ambient Light Sensor, OSD, USB 4 port hub, Plug & Play, TCO '95, Colorific Software, includes video cable.

—                           305708-001
P1610 COLOR MONITOR, NH
P1610 Color Monitor -- 24" Trinitron monitor (22.5” viewable image size), 0.25-0.28mm variable aperture grille pitch, up to 1920 x 1200 resolution @ 76 Hz, 16:10 aspect ratio, Plug & Play, TCO '95, Colorific Software, Northern Hemisphere, includes video cable.

Step 7—Color Monitors  (continued)
SN-VRQF8-24     307906-001
TFT 8000 18” PANEL MONITOR

TFT 8000 18” Panel Monitor -- 18” viewable image size, TFT active matrix, in plane switching technology 160 degree viewing angles, 1280 x 1024 resolution, brightness of 165 nits, detachable base. Requires ELSA Gloria Synergy graphics adapter.

Step 8—Network Options 

Systems include an onboard 10/100 Ethernet (Twisted pair) controller.



Maximum supported per system
Windows NT
Tru64 UNIX

SN-PCXAN-DF
388277-B21
PCI-based 10Base2 Ethernet (ThinWire) adapter 
1
1

SN-PCXAN-DE
104865-B21
PCI-based 10Base5 Ethernet (Thick wire) adapter 
1
1

SN-DE500-FA
104877-B21
PCI-based 10/100 mb/s Fast Ethernet, supports fiber MMF 
(Multi-Mode Fiber)
2
2

SN-DE500-BA
104878-B21
PCI-based 10/100 mb/s Fast Ethernet, supports Twisted pair
2
2

SN-PBXNP-AC
104879-B21
PCI-based Token Ring adapter This option must be installed in PCI-1 bus for 


both NT and Unix.
2
2

SN-DEFPA-MB
104880-B21
PCI-based FDDI adapter DAS TP-PMD
2
2

SN-DEFPA-UB
104881-B21
PCI-based FDDI adapter SAS TP-PMD
2
2

SN-DEFPA-AB
104882-B21
PCI-based FDDI adapter SAS MMF, SC
2
2

SN-DEFPA-DB
104883-B21
PCI-based FDDI adapter DAS UTP, MMF, SC
2
2

SN-PBXDA-AA
104884-B21
PCI-based 4 Port Asynchronous Controller
2
2

SN-PBXDA-AB
104885-B21
PCI-based 8 Port Asynchronous Controller
2
2

SN-PBXDP-AA
104886-B21
PCI-based 2 Port Synchronous Controller
—
2

SN-PBXDP-AB
104887-B21
PCI-based 4 Port Synchronous Controller
—
2

SN-PBXDP-AC
104888-B21
PCI-based 8 Port Synchronous Controller
—
2

SN-PBXIP-AB
104889-B21
PCI-based ISDN/BRI Controller U Interface
2
—

SN-PBXIP-AA
104890-B21
PCI-based ISDN/BRI Controller ST
2
—

SN-DGLPA-FA
104891-B21
PCI-based ATM 155mbps (fiber)- MMF (Multi-Mode Fiber)
—
2

SN-DGLPA-UA
104892-B21
PCI-based ATM 155mbps Cat5- UTP
—
2

FR-PCXFA-AA
—
ISA-based 56 KPS Internal Modem
1
1

SN-DEGPA-SA
104893-B21
Gigabit Ethernet Adapter
1
1

Step 9—Hardware Supplemental Services 

FM-AWXHW-36
3 Year 5x9 Next Business Day Onsite XP1000/667 warranty uplift (must purchase within 90 days of system purchase).

FM-AW4HR-36
3 Year 5x9 Same Day - 4 Hour Response Onsite XP1000/667 warranty uplift (must purchase within 90 days of system purchase).

FM-AW724-36
3 Year 7x24 Same Day - 4 Hour Response Onsite XP1000/667 warranty uplift (must purchase within 90 days of system purchase).

FS-CPQ05-WJ
In Warranty XP1000/667 service upgrade for 1st year to 5x9 Next Business Day

FS-CPQ05-WK
In Warranty XP1000/667 service upgrade for 2nd or 3rd  years to 5x9 Next Business Day

FS-CPQ05-35
Post Warranty XP1000/667 service – 5x9 Next Business Day

FM-AWINS-IN
XP1000/667 Hardware Installation

Step 9—Hardware Supplemental Services (continued)

Step 9a—Software Supplemental Services

For a complete list of available service offerings, please see your local Compaq sales representative or call Compaq directly at 1-800-344-4825 in the US and Canada.  For those outside North America, call +353 91 704105 for the number of your local Compaq Information Center.Windows NT

FM-NMB53-12
12 Month 5 x 9 Bronze Windows NT Software Supplemental Support Services; Tier 3M

FM-NMB53-36
36 Month 5 x 9 Bronze Windows NT Software Supplemental Support Services; Tier 3M

FM-NMB53-60
60 Month 5 x 9 Bronze Windows NT Software Supplemental Support Services; Tier 3M

FM-NMB73-12
12 Month 7 x 24 Bronze Windows NT Software Supplemental Support Services; Tier 3M

FM-NMB73-36
36 Month 7 x 24 Bronze Windows NT Software Supplemental Support Services; Tier 3M

FM-NMB73-60
36 Month 7 x 24 Bronze Windows NT Software Supplemental Support Services; Tier 3M

Tru64 UNIX

FM-3BUNS-12
12 Month 7 x 24 Bronze Software Supplemental Support Services

FM-3BUNS-36
36 Month 7 x 24 Bronze Software Supplemental Support Services

FM-3BUNS-60
60 Month 7 x 24 Bronze Software Supplemental Support Services

FM-3BUN5-12
12 Month 5 x 9 Bronze Software Supplemental Support Services

FM-3BUN5-36
36 Month 5 x 9 Bronze Software Supplemental Support Services

FM-3BUN5-60
60 Month 5 x 9 Bronze Software Supplemental Support Services

Step 10—Media and Documentation

XP1000/667 User Documentation

EK-CPWXP-SI
Compaq AlphaStation XP1000/667 Installation and Setup Guide—English

EK-?-SI
Compaq AlphaStation XP1000/667 Installation and Setup Guide—French

EK-?-SI
Compaq AlphaStation XP1000/667 Installation and Setup Guide—German

EK-?-SI
Compaq AlphaStation XP1000/667 Installation and Setup Guide—Japanese

EK-?-SI
Compaq AlphaStation XP1000/667 Installation and Setup Guide—Spanish

EK-?-SI
Compaq AlphaStation XP1000/667 Installation and Setup Guide—Italian

EK - ? 
XP1000/500 – XP1000/667 Upgrade Guide - English

EK - ? 
XP1000/500 - XP1000/667 Upgrade Guide - French

EK - ? 
XP1000/500 - XP1000/667 Upgrade Guide - German

EK - ? 
XP1000/500 - XP1000/667 Upgrade Guide - Japanese

EK - ? 
XP1000/500 - XP1000/667 Upgrade Guide - Spanish

EK - ? 
XP1000/500 - XP1000/667 Upgrade Guide - Italian

EK-?-TI
Compaq AlphaStation XP1000/667 Technical Information Guide – English only

Tru64 UNIX User Documentation, Media and Optional Licenses

For a complete list of available layered Tru64 UNIX software products see the Software Price List for Digital Products and Services document available from your local Compaq sales representative or call Compaq at 1-800-344-4825 in the US and Canada.  For those outside North America, call +353 91 704105 for the number of your local Compaq Information Center.
QA-MT4AA-H8
Tru64 UNIX Media & Documentation kit


QA-MT4AA-GZ
Tru64 UNIX Full Documentation kit

QA-MT4AB-GZ
Tru64 UNIX End-User Documentation kit

QL-MT7AE-AA
Tru64 UNIX Unlimited User License

Specifications

Height
40.64 cm (16 inches)

Width
21.59 cm (8.5 inches)

Depth
44.45 cm (17.5 inches)

Weight
18.6 kg (41 lbs)

Shipping carton 
23.38 x 20.50 x 15.36

Shipping weight
22.2 kg (49 lbs)

Compaq AlphaStation XP1000/667 System Diagram 

[image: image2.png]



Inside View Description

1,
CD-ROM drive.

2.
Front door.

3.
Floppy drive.

4.
Fan pipe (directs air to CPU heat sink area).

5.
Main Logic Board (MLB).

6.
CPU daughter card.

7.
400 Watt power supply.

8.
Internal drive bay cage.

9.
PCI option installed.

10
11

_977841905.wmf

